# Introduction to Technical Programming

# Lecturer Guide

# Module 1: Computer hardware and software

1.1 Different types of hardware of a common system

After you have completed this module, you should be able to :

- Identify the components of the system unit

- Explain the term CPU and its purpose

- Describe the term CPU and explain the impact of using various different types of CPU’s

- Describe different types of CPUs with regard to use, and power

- Define the term computer memory

- Discuss the primary purpose of memory

- Differentiate between different types of memory and their purpose (Range: RAM, ROM, CMOS, Cache memory, Flash Memory)

- Explain how data is stored on memory

- Define the purpose of the motherboard and its components

- Describe different types of input hardware with regard to use and classification e.g. direct and indirect entry

- Discuss how data is transferred between memory i.e. primary and secondary and the CPU

- Describe the Flow/transfer of data between components. (Range: USB – PnP, U3, Point-to-point connections)

- Describe the factors to consider when choosing an input device. (Range: Ergonomic considerations, Wireless vs cables)

- Describe different types of output hardware

- Describe the purpose and use of devices such as docking stations for mobile and laptop computers.

- Describe the term software

- Describe the purpose and function of software

- Describe the basic concepts of software. (Range: Software as programs. Identify software components. Concept of a graphical user interface (GUI).)

- Contrast: System software vs application software

- Differentiate between: Shareware, Freeware, Open-Source Software and Proprietary software, Firmware

- Discuss the process of how software is obtained and installed.

- Differentiate between online software and installed software.

- Discuss the following terms in relation to software. (Range: Compatibility issues, Versions, patches and service packs, Updating software)

- Launch a new Linux terminal on the Raspberry Pi

- Use the man command to get help

- Expand a Linux file path and explain each element

- List the contents of the current folder using the ls command

- Change directly location using the cd command

- Create a new folder using  the mkdir command

- Remove a folder using the rmdir command

- Remove a file using the rm command

- Rename a file using the mv command

- Copy a file using the  cp command

- Clear the command prompts screen using the cls command

- Run an executable file from the command line

# FORMATIVE ASSESSMENT 1.1 		INDIVIDUAL TASK

1.1.1	A system unit is the part of a computer that houses the primary devices that perform 	operations and produce results for complex calculations. 				(2)

1.1.2	The purpose of the CPU is to carry out the set of instructions given to the processor 	from a program.  									(2)

1.1.3	Information Processing Cycle Stages

- Fetch- Instruction from Memory (Instruction Fetch, IF)

- Decode the instructions into binary (Instruction Decode, ID)

- Execute action and move to next step or calculate address (EXE)

- Access memory operand (MEM)

- Write back result to register (WB) 						(5)

1.1.4	The rate at which one operation is completed in a second is measured in hertz. 	(1)

1.1.5	THREE main components of the CPU.

- Control Unit

- Arithmetic Logic Unit

- Registers									(3)

1.1.6	Differences between RISC and CISC

- In RISC, the instruction set is reduced, and most of these instructions are very primitive, while in CISC, the instruction set is very large that can be used for complex operations.

- RISC computer’s execution time is very less, whereas CISC computer’s execution time is very high.

- In RISC, the decoding of instructions is simple, whereas, in CISC, the decoding of instructions is complex. 							(6)

1.1.7	FOUR functions of a computer

- Data processing- Data can take many forms, and the processing requirements are numerous.

- Data storage- Even if the computer is processing data on the fly, it must temporarily store at least those pieces of data that are currently being worked on.

- Data movement- The operating environment of a computer is made up of devices that act as data sources or destinations.

- Control- A control unit manages the computer's resources and orchestrates the performance of its functional parts in response to commands.

1.1.8	The memory hierarchy is an improvement of computer storage into a hierarchy-based 	modal on response time. 							         (2)

1.1.9	Static Random Access Memory and Dynamic Random Access Memory. 	          (2)

1.1.10	Secondary memory is computer memory that is non-volatile and persistent in nature 	and is not directly accessed by a computer/processor. 				(2)

1.1.11 Hardware Devices

| Input devices | Processing devices | Output Devices |
| --- | --- | --- |
| Mouse | Motherboard | Monitor/Screen |
| Keyboard | Arithmetic Logic Unit | Printer |
| Light pen | Processor | Plotter |

(8)

Total:42 Marks

# FORMATIVE ASSESSMENT 1.2		INDIVIDUAL TASK

1.2.1	Software – set of instructions, data or programs used to operate computers and 	execute specific tasks. 								(2)

1.2.2	THREE types of language translators							(3)

Translators

Compilers

Assemblers

1.2.3	Compiler-A compiler is a computer program that transforms whole code written in a 	high-level programming language at once into the machine code whereas an 	interpreter is a computer program, which converts each high-level program statement 	into the machine code line by line. 							(4)

1.2.4

Freeware- Freeware is the software that is available to use for free of cost without any limitations.

Middleware- is software that is used to bridge the gap between applications and other tools or databases. Some examples of middleware activities include handling data and API management, authentication and messaging services.

Shareware- The software is copyrighted and distributed for free only for testing purposes. After the trial period ends, you must pay.

Open source- This is provided for use, modification, and redistribution. Open-source software is downloaded from the internet at no cost.

(8)

1.2.5	FOUR categories of system software.

- Operating System

- Device Driver

- Language Translator

- Utility Programs

1.2.6	Benefits of online software

- No software to install- Software is installed from the cloud. Once users have a license, it is easy to install. In addition, you can always retrieve the licence if lost.

- Complete flexibility on device type-Users do not need to be very careful with regards to storing the software.

- No maintenance required. - The responsibility is taken care of by the providers of the software or those who manage the cloud storage.

- Connect from anywhere at any time with internet- users can install the software from any geographical location and any time of the day.

- Centralised storage offering safe and secure data			           										          (10)

1.2.7	Compatibility is the capacity for two systems to work together without having to be 	altered to do so. 									(2)

Total:29 Marks

# FORMATIVE ASSESSMENT 1.3 		INDIVIDUAL TASK

1.3.1	To launch Linux terminal on Raspberry Pi, you need to do the following steps:

Step 1: From the Raspberry Pi icon, select Accessories

Step 2: From the Accessories, click terminal					(3) 1.3.2	ls- is used to list all files and directories in a file path on the Raspberry terminal. 	(1)

1.3.3	Raspberry Pi terminal commands

pi@raspberrypi:~/$cd Desktop

pi@raspberrypi:~Desktop $ mkdir terminal_commands

pi@raspberrypi:~/Desktop $ cd terminal_commands

pi@raspberrypi:~/Desktop/terminal_commands $ pwd

/home/pi/Desktop/terminal_commands

pi@raspberrypi:~/Desktop/terminal_commands $

pi@raspberrypi:~/Desktop/terminal_commands $ touch summative.docx

(6)

1.3.4	Adding text into the file called summative.docx

pi@raspberrypi:~/Desktop/terminal_commands $echo “Hello fellow programmers”>> summative.docx

(3)

1.3.5	Merging contents of files.

pi@raspberrypi:~/Desktop/terminal_commands $

pi@raspberrypi:~/Desktop/terminal_commands$touch formative.docx

$ echo "Terminal commands are areasy">>formative.docx

$ cat formative.docx summative.docx >> final.docx

$ tac final.docx

(8)

1.3.6 Pairing the commands to the use

| Command | Function |
| --- | --- |
| rm | This command is used to delete files within a directory. |
| rmdir | To permanently delete an empty directory. |
| man | Is used to know more about a command and how to use it |
| mkdir | Used to create a new directory |

(4)

Total: 25 Marks

# SUMMATIVE ASSESSMENT 1.4 		INDIVIDUAL TASK

1.4.1	The difference is that a Memory buffer register (MBR) contains a word to be stored in memory or sent to the I/O unit or is used to receive a word from memory or from the I/O unit. whereas Memory address register (MAR) specifies the address in memory of the word to be written from or read into the MBR. 					(4)

1.4.2	SRAM: is a memory chip that is faster and uses less power than DRAM. whereas DRAM is a memory chip that can hold more data than an SRAM chip, but it requires more power. 										(4)

1.4.3	FIVE factors to consider when choosing an input device.

- User Needs- this is the urgency of use of the device in the computer room by users also whether it will satisfy the needs of the user.

- Initial cost-the amount it can cost when buying/purchasing the devices

- Maintenance Cost-the amount that can be used to maintain the servicing of these devises should be considered.

- Mode Of Transmission- how will you transport them to the computer room should be considered.

- Compatibility With Available Hardware- will the devices fit to other devices in the room already. A device that fits your needs but doesn't work with your computer is useless.

- User- Friendliness- whether the devices will be used to solve problems and easy to be used by users							       (10)

1.4.4 Types of ROM

- MROM (Masked Read Only Memory)

- PROM (Programmable Read Only Memory)

- EPROM (Erasable and Programmable Read Only Memory)

- EEPROM (Electrically Erasable and Programmable Read Only Memory) (4)

1.4.5	Differences between microcontroller and microprocessor

- Microprocessors only have a Central Processing Unit, whereas Micro Controllers have a CPU, Memory, and I/O all integrated into a single chip.

- Microprocessors are used mainly in personal computers, whereas microcontrollers are used in embedded systems.

- Microprocessors are based on the Von Neumann model while microcontrollers are based on the Harvard architecture. 							(6)

1.4.6	Proprietary software is computer software where the source codes are publicly not 	available only the company that has created can modify it. 				(2)

1.4.7	Forward compatibility is a design principle in which a program or piece of hardware 	is designed to work with new software or devices in the future. 			(1)

1.4.8	False											(1)

1.4.9	The terminal will print the message in quotation marks and repeat the process id 	twice. Example: The process id is 590590						(2)

1.4.10	Working with terminal commands

pi@raspberrypi:~/$cd Desktop

pi@raspberrypi:~Desktop $ mkdir Practical

pi@raspberrypi:~/$cd Practical

pi@raspberrypi:~Desktop/Practical $ echo "It is the control unit that determines which machine instruction is to be executed next" >> next_file.txt

pi@raspberrypi:~Desktop/Practical $ echo " The control unit is the brains of the computer " >> next_file.tx

pi@raspberrypi:~Desktop/Practical $ pwd

(6)

Total : 40 marks

# PRACTICAL ACTIVITY					PAIR WORK

Software Installation

Search for Anydesk and install the software on your machine. You can go to this link: https://anydesk.com/en/downloads/thank-you?dv=win_exe

Share the pin numbers of the software with your friend and try controlling each other’s machine.

The software allows remote control of one’s PC. This is quite helpful if you are working with a friend or classmate, and he/she gets stuck but you are in a different geographical location. Also, if your friend or colleague is not able to follow the instructions, you can take full control of his or her computer and start assisting remotely.

# Module 2	Problem solving in computer programming

After you have completed this module, you should be able to :

- Define the term problem solving

- Define the term computational thinking

- Describe the phases of the PLDC (Program Development Life Cycle)

- Describe the purpose of problem solving leading to solutions

- Explain and apply various problem-solving steps.

- State in own words Clarity on what needs to be done

- What is known or given?

- What is missing or needed?

- Devise a plan/algorithm (storyboard – visual or

- textual)

- Look for patterns

- Look at related problems, known solutions

- Examine simpler or special cases

- Make a table, create diagram, use guess and check,

- work backwards, identify sub-goal

- Carry out the plan/implement the algorithm (write

- the code)

- Look back/test (see if it works)

- Check results against original problem. Does it make sense? Is there another solution?)

- Use appropriate tools and techniques to present a solution. Range:

- User stories (written by the client and provide the requirements)

- Noun-verb analysis of user stories

- List of nouns provides identification of objects and state

- List of verbs provides identification of behaviour

- Acceptance tests (does the program meet the

- requirements?)

- Define the term algorithm and its purpose in the problem-solving process. (Range: Basic concepts of an algorithm.

- What is an algorithm? Develop a clear understanding of the problem presented.)

- Implement and understand the basic algorithmic constructs used to create a flowchart. Range: Input, Output, Processing and Calculations, Selection, Iteration

- Create a flowchart to present a particular algorithm and its associated tasks

- Interpret a basic flow chart and describe its intended operation / function

# FORMATIVE ASSESSMENT 2.1 		INDIVIDUAL TASK

2.1.1	Problem solving is the sequential process of analysing information related to a given 	situation and generating appropriate response options.				(2)

2.1.2	Computational thinking is an interrelated set of skills and practices for solving complex 	problems, a way to learn topics in many disciplines, and a necessity for fully 	participating in a computational world. .						(2)

2.1.3	FOUR cornerstones of computational thinking are:

- decomposition

- pattern recognition

- abstraction

- algorithms 									(4)

2.1.4	Program Development Lifecyle

(6)

2.1.7	SIX steps which must followed in problem solving.

  - Understand the Problem

  - Formulate a Model

  - Develop an Algorithm

  - Write the Program

  - Test the Program

  - Evaluate the Solution

(6)

2.1.8	Write an algorithm that reads three numbers and prints the value of the largest number. 										 	           (10)

2.1.8	Algorithm

Step1: Start

Step2: Read/input A,B and C

Step3: If (A>=B) and (A>=C) then Max=A

Step4: If (B>=A) and (B>=C) then Max=B

Step5:If (C>=A) and (C>=B) then Max=C

Step6: Print Max

Step7: End

(10)

2.1.9	Algorithm is the sequence of steps to be performed in order to solve a problem by the 	computer. 										(2)

Total 32 Marks

# FORMATIVE ASSESSMENT 2.2		INDIVIDUAL TASK

2.2.1	FOUR testing phases stages of program development lifecycle

Software Testing is a method to check whether the actual software product matches 	expected requirements and to ensure that software product is defect free.

- Integration testing- The goal of integration testing is to test the interfaces between modules and expose any defects that may arise when these components are integrated and must interact with one another.

- Unit testing- Unit testing is typically performed throughout the application development process, with the goal of ensuring that every single unit or component works as expected.

- Acceptance Testing- It is a type of testing done to ensure that the users' requirements are met prior to delivery and that the software works properly in the user's working environment.

- System Testing : Complete and integrated software’s are tested in system testing, which means that all of the system elements forming the system are tested as a whole to meet the system's requirements.

- Acceptance testing is a test used to determine whether or not the requirements of a specification or contract have been met.

- Regression testing- This determines if adding additional features results in a decrease in an application's functionality.

2.2.2	Flowchart

(10)

2.2.3	SIX characteristics of a good algorithm.

- Input-must have 0 or more inputs

- Output − should have 1 or more well-defined outputs.

- Finiteness-must terminate after several steps

- Feasibility-should be feasible with available resources

- Independent-must have step by step directions independent from other 	programs									(6)

2.2.4	Average of 25 scores.

2.2.5	Algorithm to calculate area of Circle.

Step1: Start

Step 2:Input the Radius r of the Circle

Step3: Area PI*r*r // calculation of area

Step4: Print Area

Step 5: Stop

(6)

2.2.6	SIX benefits of using flowcharts.

- Communication: Flowcharts are a better way to communicate the logic of a system to all parties involved.

- Effective analysis: A flowchart can help you analyse a problem more effectively.

- Proper documentation: Program flowcharts are useful for program documentation, which is 	required for a variety of reasons.

- Efficient Coding: During the systems analysis and program development phases, the flowcharts serve as a guide or blueprint.

- Proper Debugging: The flowchart aids in the debugging process.

- Efficient Program Maintenance: The use of a flowchart simplifies program maintenance. It allows the programmer to focus his or her efforts more effectively on that aspect. 								           (12)

Total : 66 Marks

# SUMMATIVE ACTIVITY 2.3		INDIVIDUAL TASK

2.3.1	FOUR cornerstones of computational thinking are:

- decomposition - breaking down a complex problem or system into smaller, 	more manageable parts

- pattern recognition –similarities among and within problems

- abstraction – focusing on the important information only, ignoring irrelevant 	detail

- algorithms - developing a step-by-step solution to the problem, or the rules to 	follow to solve the problem							(8)

2.3.2	A conditional flowchart is a design technique used when a condition is imposed on a 	problem.									           (2)

2.3.3	Flowchart for Fahrenheit to Celsius

(5)

2.3.4 Quadratic equation

Algorithm											(7)

Step 1:	Start

Step 2:	Input a, b, c

Step 3:	Calculate d. d = sqrt(b x b – 4 x a x c

Step 4: Calculate x1. x1 = (–b + d) / (2 x a)

Step 5: Calculate x2. x2 =(–b – d) / (2 x a)

Step 6: Print x1 and x2

Step 7: Stop

Flowchart										(7)

2.3.5 Computing Interest. I=Amount * Years *Rate/100

(7)

2.2.6	THREE limitations of flowcharts

- Complex logic: The program logic can be quite complicated at times and in such instances, flowchart become more complex and clumsier.

- Alterations and Modifications: If changes are required, the flowchart may need to be completely redrawn.

- Reproduction: Because flowchart symbols cannot be typed, reproduction of flowcharts is difficult.								(6)

    - The symbol                         denotes _______

- I/O

- Flow

- Process

- Decision

2.2.8	A box that can represent two different conditions.
a) Rectangle
b) Diamond
c) Circle
d) Parallelogram

2.2.9	In computer science, algorithm refers to a pictorial representation of a flowchart.
a) True
b) False

2.2.10 The operation represented by parallelograms is ________.
a) Input/Output
b) Assignment
c) Comparison
d) Conditions

Total :46 Marks

# Module 3:Concepts of programming for single board microprocessor or microcontrollers

After you have completed this module, you should be able to :

3.1 	Introduction to IO on  single board computing

- Exploring the Arduino board

- Expand the term GPIO

- Compare and contrast some of the major advantages of python compared to other 	programming languages.

- Explain and identify where the GPIO pins are located

- Differentiate between a compiler and an interpreter

- Explain the purpose of the GPIO pins

- Discuss the major characteristics of the python programming language as an interpreted one

- Define the term physical computing

- Differentiate between a shell and an IDE

- Read and interpret a Pi GPIO Pin guide

- Define the term physical computing

- Explain what the terms 3V3, 5V, GND GP2 means on the GPIO board

- Discuss how Arduino is used to enable physical computing

3.2 Visual Programming and solution development

- Construct (code/write) using the visual tool, debug and run simple programs incorporating: Declaration of variables of different types, use and assignment of values to variables, incorporating program constructs with sequence, selection and iteration structures. Expose and apply various programming concepts as part of the coded solution such as:

- retrieving remainders: modulus

- differentiate between real value division and integer division

- comparison operators and performing logical comparisons

- incorporate and write code constructs to perform basic calculations such as area, 		volume, VAT and simple formulae, typical calculations done in other subjects

- include conditional constructs [if and ifthen-else] (up to a maximum of two 		nested levels)

- Include iteration (looping) structures [fixed counter loop]

- incorporate a combination of iteration and condition structures as part of the 	solution (i.e. program code)

- Write code which applies programming language tools and constructs to draw various shapes (turtle type commands) on an output screen/window. Reinforce concepts such 	as:

- Sequence

- Selection

- Iteration

- Creation of objects and shapes

- Design a coding solution to a problem incorporating a combination of different 	programming constructs which include:

- Sequence

- Selection

- Iteration

- Design and develop solutions for specific problems that include computational thinking and applying software engineering principles.

- Explore lists/arrays (storing and accessing a list of numbers and strings) and containers. (Range: Manipulating lists/arrays such as adding, deleting, replacing, inserting items.)

Task 3.1

Meagan bought a Raspberry Pi from takealot.com for  R2 385.09 exclusive of VAT. Calculate the VAT paid by Meagan. What was the new total price.

//declaring variables

float total_price=2385.09;

float vat_amount;

float VAT=15.00/100;

float answer;

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

}

void loop() {

//processing

Serial.print("The total price excluding VAT is ");

Serial.println(total_price);

vat_amount=VAT * total_price;

//Displaying the output

Serial.print("The amount of VAT paid is :R");

Serial.println(vat_amount);

//calculating the final amount including vat

float final_amount=total_price + vat_amount;

Serial.print("The total amount of the Raspberry Pi including VAT is :R");

Serial.println(final_amount);

delay(5000);

while ( Serial.available()==0){}

}

Output

12:59:31.859 -> The total price excluding VAT is 2385.09

12:59:31.924 -> The amount of VAT paid is :R357.76

12:59:31.956 -> The total amount of the Raspberry Pi

including VAT is :R2742.85

Task 3.2

The Quadratic Formula: x = −b ± √b²-4ac/2a

Solve: 5x2 + 6x + 1 = 0 Use Arduino  to calculate the value of  x1, x2.

//declaring variables

int a=5;

int b= 6;

int c= 1;

float x1, x2;

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

}

void loop() {

//processing

x1=(-b+sqrt((b*b)-4*a*c))/(2*a);

x2=(-b-sqrt((b*b)-4*a*c))/(2*a);

//Displaying the output

Serial.print("The value of x1 is  :");

Serial.println(x1);

Serial.print("The value of x2 is  :");

Serial.println(x2);

delay(5000);

while ( Serial.available()==0){}

}

Output

13:43:50.261 -> The value of x1 is  :-0.20

13:43:50.295 -> The value of x2 is  :-1.00

Task 3.3

Grading System.

float mark;

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

}

void loop() {

// Accepting the inputs

Serial.println("Please enter average exam mark : ");

while ( Serial.available()==0){}

mark=Serial.parseInt();

Serial.println("The mark you entered is : ");

Serial.println(mark);

//Evaluating the conditions

if(isalpha(mark)==0){

Serial.println("The character is an alphabet\n");

}

else if (mark >= 90 && mark <=100)

{

Serial.print("You got an A+ Grade");

}

else if (mark > 80 && mark <90){

Serial.print("You got an A Grade");

}

else if (mark >=70 && mark <80){

Serial.print("You got an B+ Grade");

}

else if (mark >=60 && mark <70){

Serial.print("You got an B Grade");

}

else if (mark >=50 && mark<60 ){

Serial.print("You got an C Grade");

}

else if (mark >=40 && mark<50){

Serial.print("You got an D Grade");

}

else if (mark >=30 && mark <40){

Serial.print("You got an E Grade");

}

else if (mark >=0 && mark <30){

Serial.print("You got an F Grade");

}

else if (mark <0 && mark >100){

Serial.println("Mark out of range. Please enter a mark between 0 and 100");

}

else{

Serial.print("Incorrect entry");

}

delay(5000);

while ( Serial.available()==0){}

}

Output

Please enter average exam mark :

The mark you entered is :

69.00

You got an B Grade

TASK 3.4

Print multiples of 3 between 30 and 10.

// declaring the loop variable

int x;

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

}

void loop() {

//setting up the condition staring value and end value

for (x=30; x>10; x--){

//setting the condition

if (x%3==0){

Serial.println( x);

delay(1000);

}

//if x is not divisible by 3

else{}

}

while (Serial.available()==0){}

}

Output

18:02:49.441 -> 30

18:02:50.450 -> 27

18:02:51.447 -> 24

18:02:52.450 -> 21

18:02:53.442 -> 18

18:02:54.455 -> 15

18:02:55.442 -> 12

TASK 3.5  Lighting LEDs in sequence and reverse using a while loop

Code

int timer = 300;  // The higher the number, the slower the timing.

int output_pin=6;

void setup() {

// use a for loop to initialize each pin as an output:

while (output_pin>1)

{pinMode(output_pin, OUTPUT);

output_pin--;

}

}

void loop() {

// loop from the low (pin2) to high (pin6)

while ( output_pin < 7) {

// turn the pin on:

digitalWrite(output_pin, HIGH);

delay(timer);

// turn the pin off:

digitalWrite(output_pin, LOW);

output_pin++;

}

// iterate from high (pin6) to low (pin2)

while ( output_pin >= 2) {

// turn the pin on:

digitalWrite(output_pin, HIGH);

delay(timer);

// turn the pin off:

digitalWrite(output_pin, LOW);

output_pin--;

}

}

TASK 3.6 Script to draw a hexagon

Figure 3. 1: Script for Hexagon

TASK 3.7 Drawing a triangle

Script

Figure 3. 2: Script for Triangle

# FORMATIVE ASSESSMENT 3.1		INDIVIDUAL TASK

3.1.1	Single-board computing is a complete, functioning computer in which the 	microprocessor, input/output functions, memory, and other features are all built on a 	single circuit board, with RAM built in at a pre-determined amount and with no 	expansion slots for peripherals. 							(2)

3..1.2	Types of Arduino boards

- Arduino Uno (R3)

- Arduino Nano

- Arduino Micro

- Arduino Due

- Arduino Mega (R3) Board

- Arduino Robot

ANY THREE									(3)

3.1.3	Pulse-width modulation								(2)

3.1.4	C++ vs Python								          (12)

| Parameter | C++ | Python |
| --- | --- | --- |
| Extension | C++ program are saved with .cpp extension. | Python programs are saved with .py extension. |
| Compilation | C++ is precompiled. | Python is interpreted. |
| Speed | C++ is faster once compiled as compared to python. | Python is slower since it uses interpreter |
| Nature of variables | C++ is statically typed. You can not declare a variable without a data type. | Python is dynamically typed. You do not need to indicate data type on declaration |
| Memory Management | C++ does not support automatic memory management (no garbage collector but can be implemented manually) | Python offers automatic memory management (garbage collector) |
| Functions | C++ accepts and returns a predefined type of value according to the definition. | There is no limitation on the type of the argument and the type of its return value. |

3.1.5	A compiler translates the entire source code in a single run whereas an interpreter 	translates the entire source code line by line. 					(2)

3.1.6	Binary LED Counter

Solution

Connection on the breadboard

Figure 3. 3: Binary LED Counter

Figure 3. 4: Circuit diagram form Binary LED counter

Code

//declaring the variables for the four pins

int pin1=7;

int pin2=8;

int pin3=9;

int pin4=10;

void setup() {

// setting up all the pins to OUTPUT mode

pinMode(pin1, OUTPUT);

pinMode(pin2, OUTPUT);

pinMode(pin3, OUTPUT);

pinMode(pin4, OUTPUT);

}

void loop() {

//LED Binary Counter code

//0000

digitalWrite(pin1, LOW);

digitalWrite(pin2,LOW);

digitalWrite(pin3, LOW);

digitalWrite(pin4, LOW);

delay(5000);

//0001

digitalWrite(pin1, LOW);

digitalWrite(pin2,LOW);

digitalWrite(pin3, LOW);

digitalWrite(pin4, HIGH);

delay(5000);

//0010

digitalWrite(pin1, LOW);

digitalWrite(pin2,LOW);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, LOW);

delay(5000);

//0011

digitalWrite(pin1, LOW);

digitalWrite(pin2,LOW);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, HIGH);

delay(5000);

//0100

digitalWrite(pin1, LOW);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, LOW);

digitalWrite(pin4, LOW);

delay(5000);

//0101

digitalWrite(pin1, LOW);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, LOW);

digitalWrite(pin4, HIGH);

delay(5000);

//0110

digitalWrite(pin1, LOW);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, LOW);

delay(5000);

//0111

digitalWrite(pin1, LOW);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, HIGH);

//1000

digitalWrite(pin1, HIGH);

digitalWrite(pin2,LOW);

digitalWrite(pin3, LOW);

digitalWrite(pin4, LOW);

delay(5000);

//1001

digitalWrite(pin1, HIGH);

digitalWrite(pin2,LOW);

digitalWrite(pin3, LOW);

digitalWrite(pin4, HIGH);

delay(5000);

//1010

digitalWrite(pin1, HIGH);

digitalWrite(pin2,LOW);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, LOW);

delay(5000);

//1011

digitalWrite(pin1, HIGH);

digitalWrite(pin2,LOW);

digitalWrite(pin3, HIGH);

digitalWrite(pin5, HIGH);

delay(5000);

//1100

digitalWrite(pin1, HIGH);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, LOW);

digitalWrite(pin4, LOW);

delay(5000);

//1101

digitalWrite(pin1, HIGH);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, LOW);

digitalWrite(pin4, HIGH);

delay(5000);

//1110

digitalWrite(pin1, HIGH);

digitalWrite(pin2,HIGH);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, LOW);

delay(5000);

//1111

digitalWrite(pin1, HIGH);

digitalWrite(pin2, HIGH);

digitalWrite(pin3, HIGH);

digitalWrite(pin4, HIGH);

delay(5000);

}

(20)

[Total =41 marks]

# FORMATIVE ASSESSMENT 3.2		INDIVIDUAL TASK

3.2.1	A visual programming language (VPL) is a computer program that develops 	applications using graphical components and figures.				(2)

3.2.2	FOUR common examples of visual programming languages

- Scratch

- Ardublock

- mBlock

- Minibloq.

ANY CORRECT FOUR 								(4)

3.2.3	Arduino Traffic Light Project using Pictoblox to depict the scenario and Arduino 	Uno.

Sample connections

3.2.3

You need :

- Breadboard

- 3 LEDs (Green, Red, Orange)

- 3 x 220-ohm resistors

- Jumper Cables

Follow the connections on Figure 3.15.

CODE STRUCTURE

(10)

3.2.4	A variable is a place to store data and values in memory location. Variables consist of 	a name, value, and type.							           (2)

3.2.5	Real numbers can include fractions due to rational and irrational numbers while integer 	division divides two numbers and returns the integer part of the result. 		(4)

3.2.6	Programming constructs

- Sequence construct

- Selection construct

- Iteration construct								(3)

[Total =25 marks]

# SUMMATIVE ACTIVITY 3.3		INDIVIDUAL TASK

3.3.1	An Arduino is a microcontroller and a Raspberry Pi is a microprocessor.		(2)

3.3.2	Pins on an Arduino  Uno board							(3)

- 14 Digital pins

- 6 Analogue pins

- Power Pins

3.3.3	Key attributes of computational thinking.

  - Decomposition

  - Pattern recognition

  - Abstraction

  - Algorithms									(4)

3.3.4	void setup(){}- The commands put inside the curly brackets will be executed once. For 	example the pins used in the program can be declared in here and  whereas the code in 	the void loop() will be executed continuously repeated constantly.			(4)

3.3.5	pinMode() defines how the pins of the Arduino are to work either as INPUT or 	OUTPUT mode.

Syntax

pinMode(pin, Mode)

Example

pinMode(11, OUTPUT)								(4)

3.3.6	digitalWrite() sets a digital pin to either HIGH or LOW, while analogWrite() sets the 	voltage of a Pulse Width Modulation pin.						(4)

3.3.7	To upload code to an Arduino board, you must connect the device to your computer 	using a USB cable and then  you can use the Arduino IDE to upload your code to the 	board.											(2)

3.3.8

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

}

void loop() {

Serial.println("Arduino count down for loop");

for (int i=10; i>=1; i--) {

Serial.print("i is : ");

Serial.println(i);

}

delay(500);

while ( Serial.available()==0){}

}

(2)

3.3.9	Displaying decimal equivalent of a Binary number. (Binary to decimal 	convertor.

(5)

3.3.10	Use an Pictoblox to calculate area of a rectangle whose length and width  is entered 	by the user.

[Total = 34 marks]

# Module 4	Programming tools and utilities

After you have completed this module, you should be able to :

- Define the term compiler

- Define the term source code

- Explain what a decompiler is used for

- Define the term interpreter

- Explain the difference between a compiler and an interpreter

- Explain what a binary is and when it is produced

- Explain the difference been C and C++ compiler

- List and Explain the basic three stage compiler design (Front Middle Back end)

- Install and configure C/C++ compiler on desktop PC

- Define the term IDE

- Explain what an IDE is used for

- Install and configure IDE on desktop PC

- Explain what the acronym GCC stands for

- Explain what the GCC collection contains

- List compilers included in the GCC

- Explain the difference between GCC and MinGW

- Install and configure GCC C/C++ compiler on Raspberry Pi

- Install and configure Geany on Raspberry Pi

- Define the term debugging

- Explain why an application needs to be debugged

- List common debugging techniques (Interactive, Print, Remote)

- Define the term breakpoint as it relates to debugging

- Define the term stepping as it relates to debugging

- Explain what the GDB tool is used for

- Install and configure MinGW on desktop PC

- Locate the GDB application in the MINGW installation

- Add the MINGW bin directory to the operating system path environment variable

- Compile source code wit the -g flag in IDE

- Explain the concept of a debugger frontend

- Install and configure gdbgiu on Desktop PC

- Start gdbgui application

- Load a compiled C/C++ binary (.exe)

- Run the loaded binary

- Step through the running binary line by line

- Investigate (watch) the values of local variables

- Explain what Git is and what Git is used for

- Discuss the three main goals of the Git VCS

- Explain the relationship between local Git directory and directory located on server

- Discuss the software license used for Git

- Name and explain the advantages and disadvantages to the software license used by Git

- Explain why Git was created

- Explain the relationship between Git and hosting providers like GitHub

- List major open-source projects using Git

- Install and configure Git on desktop PC

- Install and configure Git on Raspberry Pi

- List and explain common terminology associated with distributed VCS’s

# FORMATIVE ASSESSMENT 4.1		INDIVIDUAL TASK

4.1.1	A compiler is a language processor that reads a whole source program written in high-	level language in one go and converts it into an equivalent program written in 	machine code.										(2)

4.1.2	Compiled languages.

- C, C++

- C#

- Java

- Erlang

- Haskell

- Rust

- Go.

ANY OF THE THREE								(3)

4.1.3	The back end of compiler includes those portions that depend on the target machine and generally those portions do not depend on the source language, just the intermediate language. These include

1. Code optimization

2. Code generation, along with error handling and symbol- table operations.

(3)

4.1.4	Advantages of a compiler

Advantages of compilers

- Independence-No other program or application is required to run the 	executable file of source codes.

- Optimisation-The compiled program is well optimized and runs faster.

- Data Security-The compiler generates executable files that can be executed on 	any 	other system.

- Speed-Compiler are faster than interpreter

Disadvantages of Compilers

- Hardware Specific

- Time consuming- Compilation process takes time before an executable file is produces

- Extra memory-Because source code object code file and executable files

- Debugging Difficulty- All errors are shown at once.				(6)

4.1.5	An interpreter program executes other programs directly, running through program code and executing it line-by-line.							(2)

4.1.6	Differences between compiler and interpreter.

| Compiler | Interpreter |
| --- | --- |
| A compiler translates the entire source code in a single run. | An interpreter translates the entire source code line by line. |
| Compilers are faster than an interpreter. | Interpreters are slower than the compiler. |
| CPU utilization is more. | CPU utilization is less as compared to the compiler. |
| Both syntactic and semantic errors can be checked simultaneously. | Only syntactic errors are checked. |
| The compiler is larger than interpreter therefore require more memory. | Interpreters are often smaller than compilers. |
| The localization of errors is difficult. | The localization of error is easier than the compiler. |

(12)

Total : 28 Marks

# FORMATIVE ASSESSMENT 4.2		GROUP TASK

4..2.1	As part of software installation, you are expected to be able to install any IDE and the C++ compiler. We have listed a lot of IDE’s such as Code::Blocks, Geany, CLion, VSCode.

Task.

In groups, you are required to uninstall Code::Blocks and Mingw.

Reinstall Code::Blocks and Mingw

Test  if gcc/g++ are available.

Run a simple hello world program.

(20)

# FORMATIVE ASSESSMENT 4.3		INDIVIDUAL TASK

4.3.1	GCC stands for “GNU Compiler Collection”. GCC is an integrated distribution of compilers for several major programming languages such as  C, C++, Fortran, Ada, D, and Go.										(2)

4.3.2	GCC stands for GNU Compiler Collection -on open-source collection of compilers for various languages based on a common core compiler and linker whereas MinGW stands for Minimal GNU for Windows. This is a copy GCC plus other common utilities and libraries used in compiling and linking C or C++ code using the GNU tools.	(2)

4.3.3	An integrated development environment (IDE) is a software application that provides 	comprehensive facilities to computer programmers for software development. 	(2)

4.3.4	Installing Geany on a Raspberry Pi.

Step 1:	Navigate to the Raspberry Icon

Step 2: Select  accessories and select terminal.

Step 3: Type the following command:

sudo apt-get install geany

Step 4: Press enter. Geany IDE will be installed.

(4)

4.3.5	Complete the following table to differentiate G++ from GCC.			(8)

| Parameter | G++ | GCC |
| --- | --- | --- |
| Uses |  |  |
| Compilation |  |  |
| Compilation Command |  |  |
| File Linking in Library |  |  |

4.3.5	Differences between G++  and GCC

| Parameter | G++ | GCC |
| --- | --- | --- |
| Uses | G++ is used for compiling C++ | Used for compiling C |
| Compilation | can compile either.cpp or.c files, but they will only be treated and released as C++ files. | can compile either.cpp or.c files, but they will be treated and released as C++ or C |
| Command for Compilation | g++ fileName.cpp -o binary. | gcc fileName.c -o binary. |
| File Linking in Library | When we use the G++ command to link the object files, the files automatically link in the standard C++ libraries. | It does not happen in the case of GCC. |

(8)

Total : 18 marks

# FORMATIVE ASSESSMENT 4.4		INDIVIDUAL TASK

4.4.1

a) 	Syntax errors- is a bug that occurs when a computer program has an 			incorrectly typed statement.

b)	Runtime errors-is a type of error which during runtime and could be caused by 		inputting wrong data type.

c)	Logic errors-occurs when program runs but gives incorrect results.		(6)

4.4.2	FOUR reasons for debugging computer programs.

a) 	Debugging ensures that the final product is bug free.

b)	Competitive advantage-Bug free programs will build a good reputation for the 	software firm.

c)	Saves Time- Performing debugging at the initial stage saves the time of 	software developers.

d) 	Debugging ensures that requirements are met.				(4)

4.4.3	Remote debugging technique.								(1)

4.4.4	Stepping is a debugging technique that involves executing code one instruction or line 	at a time.										(2)

4.4.5	Setting up a breakpoint

Figure 4. 1: Sample code

Output.

Figure 4. 2:Output

(10)

Total: 22 marks

# FORMATIVE ASSESSMENT 4.5		GROUP TASK

4.5.1	Installing gdbgui and debugging a simple program

Make use of the following program.

This section will need to be repeated again once students master big programs, debugging very small programs at this point might not show the intended purpose.

# FORMATIVE ASSESSMENT 4.6		INDIVIDUAL TASK

4.6.1	Version control is a system that records changes to a file or set of files over time so 	that you can recall specific versions later.						(2)

4.6.2	Git is a free and open-source distributed version control software designed to handle 	everything from small to very large projects with speed and efficiency whereas 	GitHub is a web-based Git repository hosting service, which offers all of the 	distributed revision control and source code management (SCM) functionality of Git.

(4)

4.6.3	FIVE benefits of using Git.

- Data replication and redundancy are both possible.

- It is a service with high availability.

- There can only be one Git directory per repository.

- Excellent network and disc performance are achieved.

- On any project, collaboration is very simple.

4.6.4	$ git init                                            						(2)

4.6.5	Git -v or git –version or git version gives the version of git software installed on the machine (pc or laptop).								(2)

4.6.6	Adding files to Git

Sample Solution

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop

$ mkdir git_revision

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop

$ cd git_revision

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop/git_revision

$ echo "Working with git and github is saves time" >> file1.txt

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop/git_revision

$$ git init

Initialized empty Git repository in C:/Users/gdzvapatsva/Desktop/git_revision/.git/

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop/git_revision (master)

$ git add .

warning: in the working copy of 'file1.txt', LF will be replaced by CRLF the next time Git touches it

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop/git_revision (master)

$ git commit -m "initial push"

[master (root-commit) 59dc420] initial push

1 file changed, 1 insertion(+)

create mode 100644 file1.txt

- gdzvapatsva@GDzvapatsva-LP MINGW64 ~/Desktop/git_revision (master)

$ git remote add origin https://github.com/gpdzvapatsva/git_revision.git

- git branch -M main

- git push -u origin main

To https://github.com/gpdzvapatsva/git_revision.git

* [new branch]      main -> main

branch 'main' set up to track 'origin/main'.

(20)

Total :35 marks

# SUMMATIVE ACTIVITY 4.7		INDIVIDUAL TASK

4.7.1	The fundamental distinction between distributed and centralized version control is that although versions in distributed version control can be saved both in local machine repositories and remote repositories, versions in centralized version control are saved in the remote repository only.					             		(4)

4.7.2	Source code is the term used to describe programming statements that are created by a programmer using a text editor or visual programming tool and then saved in a file.(2)

4.7.3	FOUR examples of IDE’s used for C++ programming.			             (4)

- Geany

- VSCode

- Code::Blocks

- CLion

- CodeLite

4.7.4	C++ uses cout command to display output to the screen.				(2)

4.7.5	Advantages of using GIT.

- When using DVCS, all contributors can work concurrently

- Allows programmers to work remotely

- Speeds up development process						(3)

4.7.6	git clone https://github.com/gpdzvapatsva/git_revision.git				(2)

4.7.7	The git config list command will display every Git config property found in every Git 	file with a different scope.								(2)

4.7.8	GNU Compiler Collections, or GCC, is used to compile primarily C and C++ code. 	The g++ command invokes the GNU C++ compiler and is used to prepare, compile, 	assemble, and link source code to create executable files. G++ is primarily used to 	compile C++ while gcc is used for C programs. G++ links the object files in std 	libraries while gcc does not do the same.						(4)

4.7.9	git config --global user.name "cplusguru"						(2)

4.7.10	THREE common debugging techniques

- Print debugging

- Interactive debugging

- Remote debugging 								(3)

Total :28 Marks

# Module 5: Introduction to a high-level programming language

By the end of the module, students should be able to:

- Create a new project using an IDE

- Create a new file with extension CPP

- Explain why C++ source files use the .cpp extension and C++ source files uses the .c extension.

- Write the C++ code to display output “Hello World”.

- Compile a C++ application using an IDE

- Explain what type of file is produced by the C++ compilation process

- Show or hide the IDE message window

- Investigate the IDE message window to determine if compilation succeeded

- Investigate the IDE message window to locate line numbers that contain errors

- Build a C++ application using an IDE

- Explain what file type is produced by the C++ build process

- Run a C++ application using the IDE

- List and explain the different parts of a simple C/C++ application

- Explain case sensitivity as it applies to C/C++

- Explain the term free-format language and how that applies to C/C++

- Explain the term block-structured language and how that applies to C/C++

- Explain what a comment is used for in the C/C++ language

- Explain how the compiler will treat a comment

- List and explain two types of comments allowed by the C/C++ compiler

- Discuss the importance of adding comments to a C/C++ application

# FORMATIVE ASSESSMENT 5.1 		INDIVIDUAL TASK

5.1.1	High-level languages enable programmers to write instructions in a more 	understandable language than low-level languages. Examples of high-level languages 	includes Python, C++, Java etc.							(2)

5.2.2	An integrated development environment (IDE) is a software suite that consolidates 	basic tools required to write and test software. Examples of IDE’s which can be used 	for C++ are Code::Blocks, Geany, VSCode etc.					(2)

5.2.3	Extension for C++ is .CPP.								(2)

5.2.4	Compiling a C++ program entails converting the source code (.cpp files) into an 	executable or library that can run on a specific platform. This process can be divided 	into three key stages:

- Pre-processing-The outputted code will be ready to compile by the end of the pre-processor stage.

- Compilation-C++ compilation is a two-step procedure. First, the compiler converts the source code to assembly language. Second, using an assembler, the assembly language is converted into actual machine code. The resulting output is a collection of files known as an object file. The object code file has .obj or .o file extension

- Linking-The final stage generates the executable or library.

(4)

5.2.5	Structure of C++ program

(10)

5.2.6	C:\Users\gdzvapatsva\Desktop\exam>notepad

Type the following

#include <iostream>

using namespace std.

int main (){

/*This is the main block

I am learning about multiline comments

*/

cout<<"Compiling C++ using command line”.

return 0.

}

- Save as exams.cpp

Navigate to exams folder on desktop and enter the following commands

- g++ -g exams.cpp -o exams1.exe

- exam>exams1								           (10)

5.2.7	Compiling C++ on Geany

- Build

- Compile

- Run										(3)

5.2.8	Compiling is the process of converting the high-level source code files to machine level code whereas Building is the process of converting the high-level language to a executable. It would involve compiling and linking .					(4)

Total: 34 marks

# FORMATIVE ASSESSMENT 5.2 		INDIVIDUAL TASK

5.2.1

- Preprocessor directive- Preprocessor directives are lines included in the code of programs preceded by a hash sign (#).They examine the code before actual compilation of code begins and resolves all these directives before any code is generated by regular statements. Some of the examples are #define, #include.	(2)

- Header - Header files contain definitions of function and data types; these header files are imported into any C++ program using the preprocessor directive #include. The preprocessor directive tells the compiler to process these files before compilation. Example of a header file is <iostream>, <fstream>, <cmath.h>, <complex.h>.The .h extension is not mandatory.				(2)

- Namespace- Namespaces give us a place to define or declare identifiers, such as variables, methods and classes. Namespaces are used to organize code into logical groups and to prevent name collisions, which can occur when your code base includes multiple libraries.							(2)

5.2.2	C++ Program to print two lines.

#include <iostream>

using namespace std;

int main()

{

cout << "\n\n Print a welcome text in a separate line :\n";

cout << "----------------------------------------------\n";

cout << " Welcome to NCV programming\n" ;

cout << " Developing future coders for the digital world"<<endl ;

return 0;

}

(8)

5.2.3	Debugging Solution

#include <iostream>

using namespace std;

namespace addition

//This is a block of code

{

void add_numbers(){

cout << " The sum of 29 and 30 is : "<< 29+30 <<"\n\n" ;

}

}

int main()

{

addition::add_numbers();

}

5.2.4	True or False

a)	Since C is a subset of C++, all C programs will run under C++ compilers. False

b)	By merely looking at one or two lines of code, we can easily recognise whether a program is written in C or C++.  False. Most lines are the same in C or C++

c)	The main {} function in C++ and in C are the same. False. C++ returns integer by 	default while C returns void.						(3)

5.2.5	Comments in C++ are simply a piece of source code ignored by the compiler.  They 	help the programmer to add additional information to source code.			(2)

5.2.6	C++ comments

- //single-line comment

- /* block comment */								(4)

5.2.7	A free-format language is a programming language in which character positioning on the page in program text is irrelevant. Program text does not need to be placed in specific columns. C++ is a free-format programming language.			(2)

Total:30 Marks

# SUMMATIVE ACTIVITY 5.3		INDIVIDUAL TASK

5.3.1	Steps to start a new project in Geany

Step1:	Open Geany from the Taskbar

Step 2: Click on File and select New

Step 3:	Click on Document then Set FileType and select programming languages. 			Choose C++ source file

Step 4:	Click File and select Save As. Assign a new name addition.cpp

Step5:	Type the simple program

Step 6: Click Build and select Compile

Step 7: Click Build and select Build

Step 8: Click Build and select Execute						(8)

5.3.2	True											(1)

5.3.3 Solution

#include <iostream>

using namespace std;

namespace languages

//This is a block of code

{

void display(){

cout << " C++ is an extension of C "<<endl;

}

}

namespace jobs

{

void display(){

cout << " I want to work as a backend developer";

}

}

int main(){

languages::display();

jobs::display();

}

(5)

5.3.4	Object files with .O extension are produced at compilation stage.			(2)

5.3.5

- Compile-time: These occur due to misuse of language constructs, such as syntax errors. Normally easy to find by using compiler tools and warnings to fix reported problems.

- Run-time: These are much harder to figure out, as they cause the program to generate incorrect output (or “crash”) during execution. Example is a runtime error.

(4)

5.3.6	A block-structured programming language is one that allows the creation of blocks, 	including blocks nested within other blocks.	C++ define blocks of scope using some 	sort of delimiter pair for e.g., braces in {  } to denote a method.

namespace jobs

{

void display(){

cout << " I want to work as a backend developer";

}

(4)

5.3.7	Importance of comments

- Saves time- new programmers who join the organisation can quickly understand the code segments by reading comments.

- Comments helps to identify important blocks of code easily.

- Comments help to understand why programmers used a certain way to get a solution rather than the other option.

- Comments help to add description and clarity of what certain blocks of code do by adding context.								(4)

5.3.8	A breakpoint is a deliberate halting or pausing point placed in a program for 	debugging purposes in software development.				           (2)

5.3.9 	A syntax error is a bug that occurs when a computer program has an incorrectly typed statement. Here is an example of incorrectly typed reserved words which will generate syntax errors.

Line 2 and 4 have errors in spelling of reserved words namespace has no space in between.

Line 4 the correct spelling is cout.

(3)

Total :31 Marks

# Module 6: Data Types, Variables and Output

After you have completed this module, you should be able to :

- List Data Types in C/C++

- Define the term data type

- Define the term literal value

- Name and explain two numerical data types used in C/C++

- Explain the difference between Integer data types and floating-point data types

- List the different integer data types

- List the different floating-point data types

- Explain the float, double data types in terms of:

- What the type can consist of

- What the maximum and minimum value can be for the type

- What operations is allowed on the type

- Example literal values allowed for the type

- Example literal values not allowed for the type

- Whether the type is signed or unsigned

- Number of bytes memory used to store type value

- Explain the int, char, bool, float double data types in

terms of:

- What the type can consist of

- What the maximum and minimum value is

- What operations is allowed on the type

- Example literal values allowed for the type

- Example literal values not allowed for the type

- Whether the type is signed or unsigned

- Number of bytes memory used to store type value

- Use variables as part of a solution

- Define the term variable

- Explain how variables are used in C/C++ applications

- Identify and list C/C++ key words

- List and describe the syntax rules for naming variables

- Write C/C++ code to create variables of different types

- Write C/C++ code to assign literal values to variables of different types

- Write C/C++ code store the result of an arithmetic expression (simple and complex) in a variable

- Write C/C++ code to print the value associated with a variable (combined with string literal for context)

- Write C/C++ code store the result of an arithmetic expression that contains variables on both sides of the operand (simple and complex) in a variable

- Write C/C++ code to assign the value of one variable to a second variable

- Explain the difference between variable declaration and variable initialisation

- Write C/C++ code to initialise variables of different types

- Write C/C++ code to increase or decrease the value of a variable (accumulation statements)

- Write C/C++ code to increase or decrease the value of a variable with 1 using the increment

- Operator

- Write C/C++ code to print the value associatedwith a variable (combined with string literal for context)

- List the different arithmetic operators

- Define the term binary operator

- Define the term operand

- List and explain (with examples) what a binary arithmetic expression consists of

- Write C/C++ code that uses printf or cout to display the value of different arithmetic expressions (combined with string literal for context)

- List and explain the data type produced based on the data type of the operands involved

- List and explain the rules involved when creating complex arithmetic expressions containing multiple operands.

- List and describe the three levels of precedence.

- Write C++ code that uses printf or cout to display the value of different complex arithmetic expressions (combined with string literal for context)

- Identify or correct the general form for multiple variable declarations

- Explain why multiple declarations should be of the same type

- Write C/C++ code that declares multiple variables in a single line

- Escape characters and New Lines

Escape characters

- Explain what meaning the backslash character will have in C/C++

- Define the term escape character

- Define the term escape sequence

- Write C/C++ code that uses cout to display/use variable (combined with string literal and variable values for context):

- Backslash

- Question mark

- Single and Double quotation

- Horizontal tab

- Alert

- Give examples of where output in an application could be formatted to be more attractive to the user

- Explain what field with manipulators are used for

- Write C/C++ code that uses printf and formatting strings to:

- Set the field width

- Set the default leading fill character

- Left justify numbers

- Right justify numbers

- Set floating point precision

FORMATIVE EXERCISES				GROUP/INDIVIDUAL TASKS

Task 6.1	: Program to show the following output making use of sizeof keyword

Solution

#include <iostream>

using namespace std;

int main()

{

//printing the different size of data types

cout << "\n\n The following are the sizes of different data types :\n";

cout << "------------------------------------------\n";

cout << " The sizeof(char) is      :" << sizeof(char) << " bytes \n" ;

cout << " The sizeof( short) is    :" << sizeof(short) << " bytes \n" ;

cout << " The sizeof(int) is       :" << sizeof(int) << " bytes \n" ;

cout << " The sizeof(long) is      :" << sizeof(long) << " bytes \n" ;

cout << " The sizeof(long long) is:"<< sizeof(long long) << " bytes \n";

cout << " The sizeof(float) is     :" << sizeof(float) << " bytes \n" ;

cout << " The sizeof(double) is    :" << sizeof(double) << " bytes \n";

cout << " The sizeof(long double) is:" << sizeof(long double)<<"bytes \n";

cout << " The sizeof(bool) is      :" << sizeof(bool) << " bytes \n\n";

return 0;

}

Task 6.2	: output of binary literal addition

Solution

sum is 3

TASK 6.3	:Evaluating a quadratic equation and find the roots (x1,x2).

0b110x 2 – 0b110000x – 0b110110 = 0 a=6, b=48, c=54

Solution

#include <iostream>

#include <cmath>

using namespace std;

//declaring variables

float x1, x2;

int main()

{

// 0b110x2 – 0b110000x − 0b110110 = 0 a=0b110, b=0b110000, c=0b110110

//processing

x1=(-(-0b110000)+ sqrt((0b110000*0b110000)-4* 0b110*-0b110110))/(2*0b110);

x2=(-(-0b110000)-sqrt((0b110000*0b110000)-4* 0b110*-0b110110))/(2*0b110);

//displaying the output

cout << "The value of x1 is :"<<x1 << endl;

cout << "The value of x2 is :"<<x2 << endl;

return 0;

}

Output

The value of x1 is :9

The value of x2 is :-1

TASK 6.4	:Program in C++ to swap two numbers.

Solution

#include <iostream>

using namespace std;

int main()

{

cout << "\n\n Swap two numbers :\n";

cout << "-----------------------\n";

int num1, num2, temp;

cout << " Input 1st number : ";

cin >> num1 ;

cout << " Input 2nd number : ";

cin >> num2;

temp=num2;

num2=num1;

num1=temp;

cout << " After swapping the 1st number is : "<< num1 <<"\n" ;

cout << " After swapping the 2nd number is : "<< num2 <<"\n\n" ;

Task 6.5	:Using a ternary operator to write a program to check if a person qualifies 		to vote or not.

SOLUTION

#include <iostream>

using namespace std;

int main()

{

int age;

cout << "Please enter your age: !" << endl;

//entering age

cin>>age;

//ternary condition to check age

string result = (age >=18 ) ?  " can vote "  : " can not vote";

//displaying output

cout<<"Based on the analysis of your age, your " <<result;

return 0;

}

Output

Please enter your age: !

15

Based on the analysis of your age, your  cannot vote

TASK 6.6	: Program to print the sum of two numbers using variables. The program 		must 	ask user to input the values of the two numbers and display the 		output as:

The sum of 67 and 23 is : 90

Solution

#include <iostream>

using namespace std;

int main()

{

//declaring variables

int a=0, b=0, sum=0;

cout << "\n\n Program to print the sum of two numbers :\n";

cout << "-----------------------------------\n";

//accepting input

cout << "Enter the value of a\n";

cin>>a;

cout << "Enter the value of b\n";

cin>>b;

//processing

sum=a+b;

//displaying output

cout << " The sum of "<< a << " and "<<b <<" is : "<< sum <<"\n\n" ;

return 0;

}

Output

Enter the value of a

67

Enter the value of b

23

The sum of 67 and 23 is : 90

TASK 6.7

Simplify 4 + (2 + 1)2.

4 + (2 + 1)2

= 4 + (3)2

= 4 + 9

= 13

Task 6.8	:Program to compare the following complex numbers.

complex_two(3.14, 1.5);

complex_three(1.5, 3.14);

Result

#include <iostream>

#include <complex>

using namespace std;

int main()

{

//declaring the complex number variables and assigning values

complex<double> complex_two(3.14,1.5);

complex<double> complex_three(1.5, 3.14);

//comparing complex numbers

bool result=(complex_three==complex_two);

//displaying output

cout <<result<<endl;

return 0;

}

Output

0

Task 6.9	:Identifying errors

Correct the program to produce the following output:

Cape Town temperature is 26 degrees celcius

Joburg temperature is 26 degrees celcius

Durban temperature is 26 degrees celcius

Solution

#include <iostream>

using namespace std;

int main()

{

//declaring multiple variables

float cpt_temp, jbg_temp, durb_temp;

cpt_temp=jbg_temp=durb_temp=26;

cout<<"Cape Town temperature is "<<cpt_temp<<"degrees celcius"<<endl;

cout<<"Joburg temperature is "<<jbg_temp<<"degrees celcius"<<endl;

cout<<"Durban temperature is "<<durb_temp<<"degrees celcius"<<endl;

return 0;

}

Task 6.10	:Program to compute quotient and remainder.

Solution

#include <iostream>

using namespace std;

int main()

{

//declaring the variables

int dividend, divisor, quotient,remainder;

cout << "\n\n Calculating quotient and remainder :\n";

cout << "-------------------------------------\n";

//capturing input

cout<<" Input the dividend : ";

cin>>dividend;

cout<<" Input the divisor : ";

cin>>divisor;

//processing

quotient=dividend / divisor;

remainder=dividend % divisor;

//displaying the result

cout<<" The quotient of the division is : "<< quotient << endl;

cout<<" The remainder of the division is : "<< remainder << endl;

cout << endl;

return 0;

}

Output

Calculating quotient and remainder :

-------------------------------------

Input the dividend : 35

Input the divisor : 4

The quotient of the division is : 8

The remainder of the division is : 3

Task 6.11	:Program to show the different values of an integer using the setbase() 			function.

Solution

#include <iomanip>

using namespace std;

int main () {

int x = 12349;

cout.fill('=');

cout.width(15); cout << left <<x<< '\n';

cout.fill('=');

cout.width(50);   cout <<  left<<"Octal value is: " << setbase(8) << x << endl;

cout.fill('*');

cout.width(50); cout <<right<< "Hexadecimal value is: " << setbase(16) << x << endl;

return 0;

}

# FORMATIVE ASSESSMENT 6.1		INDIVIDUAL TASK

6.1.1	A data type specifies the type of data that a variable can store such as integer, 	floating, character etc.									(2)

6.1.2	Three categories of data types in C++

- Primitive/Built-in data types e.g int, float, char, wide character, Boolean, double, void

- Abstract data types e.g functions, Array, Pointers and Reference data types

- Derived data types e.g class, structure, union, enumeration, Typedef defined data types											(9)

6.1.3	Literals are data used for representing fixed values e.g integer literals, boolean literals, 	float literals.

6.1.4	Representing decimal to the different number literals

| Decimal Number | Type of integer literals | Example |
| --- | --- | --- |
| 45 | Binary-literal (base 2) | 0b101101 |
| 87 | Binary-literal (base 2) | 0B1010111 |
| 45 | Octal-literal (base 8) | 055 |
| 124 | Octal-literal (base 8) | 0174 |
| 197 | Hexadecimal literal(base 16) | 0x C5 |
| 350 | Hexadecimal literal(base 16) | 0x 15E |

(6)

6.1.5	c) Explanation: The const will declare with a specific type of value and #define is 	used to declare user defined constants.						(2)

6.1.6	B.	Explanation: Logical values can be either true or false, so the boolean type is 		suited 	for it.									(2)

6.1.7	Conversion error. Data is a string.							(2)

6.1.8	Sizeof is a unary compile-time operator used to determine variables, data types, and 	constants' sizes in bytes.								(2)

6.1.9	A variable is a place to store data and values in memory location. 			(2)

[Total=27 Marks]

# FORMATIVE ASSESSMENT 6.2		INDIVIDUAL TASK

6.2.1	Multiple variables

int x,y,z;

x=y=z=53;

OR

int x=53,y=53,z=53;

cout<<z<<endl;									(2)

6.2.2	An operator is a symbol that operates on a value to perform specific mathematical or 	logical computations.									(2)

6.2.3	THREE categories of operators							(3)

- Unary Operators

- Binary Operators

- Ternary Operators								(3)

6.2.4	A binary expression is one in which there are two or more operands and one or more 	operators. Operands are the values which are acted upon by the operator for example.

answer= num2 + num3

num2 and num3 are the operands; + is the operator.					(4)

6.2.5	Multiple declarations.

int num2=35, num3=17, answer=0;

answer= num2 + num3;

cout<<num2 << " + " << num3 <<" = "<< num2+num3<<endl;

(6)
6.2.6	Program to calculate area and perimeter of circle.

#include <iostream>

int main()

{

using namespace std;

//declaring variables

float perimeter=0, area=0, radius=0;

//declaring a constant pi

const float pi= 3.14;

cout << "Enter radius of circle" << endl;

//accepting input

cin >> radius;

//calculating perimeter

perimeter = 2*pi*radius;

//calculating area

area=pi*radius*radius;

//displaying results

cout << "Perimeter whose radius is " <<radius<<" is " <<perimeter<<"cm2"<< endl;

cout << "Area whose radius is " <<radius<<" is " <<area<<"cm2"<< endl;

return 0;

}

(9)

6.2.7	Comparing values using comparison operators and logical operators.

#include <iostream>

int main()

{

using namespace std;

int first_number;

int second_number;

cout<<"Please enter the value of first number"<<endl;

cin >> first_number;

cout<<"Please enter the value of second number"<<endl;

cin >> second_number;

//relational and logical operators

cout << ((first_number < 50) && (first_number < second_number) )<< endl;

return 0;

}

The output is 1 (true)								(10)

6.2.8	Casting is a conversion process wherein data can be changed from one type to 	another.  										(2)

6.2.9	Output

Output 1 is 4

Output 2 is1

Output 3 is 1

Output 4 is 0

Output 5 is 0

Output 6 is 1

Output 7 is 0

Output 8 is 0

Output 9 is 1

Output 10 is 1

(10x1=10)

[Total =51 Marks]

# FORMATIVE ASSESSMENT 6.3		INDIVIDUAL TASK

6.3.1	Variable naming rules

- Variable names must not start with a number

- Variable names can consist of alphanumeric characters.

- Special characters are not allowed when naming variables. Underscore is permitted.

- No use of keywords in variable names

- Variables must have unique names, i.e., you cannot have two variables with the same name.

- Variable names are case sensitive, so firstname and FIRSTNAME are two different variables.

ANY CORRECT FIVE								(5)

6.3.2 Operator precedence

- Answer is  4

- Answer is 0

- Answer is 3

- Answer is -2									(8)

6.3.3	It is important so that there are no space allocation issues. In fact, any attempt to use 	different data types as follows will result in compilation errors.

int a = 1, float b = 2, c = 3, d = 4;					(2)

6.3.4

amount = 2 * count++ -count ;

The output is 2

The code is evaluated as follows:

Amount=(2*count++)-count

Amount=6 -4  (We used 4 since the value of count was incremented though the original value was the one used in the expression within the bracket.					(3)

[Total =18 marks]

# FORMATIVE ASSESSMENT 6.4				INDIVIDUAL TASK

6.4.1

cout << " The town of \"Mthatha\" is in the Eastern Cape "<<"\n" ;

(2)

6.4.2	Escape sequences in C++ are character combinations that comprise a backslash (\) followed by some character or symbol. 						(2)

6.4.3

#include <iostream>

#include <iomanip>

using namespace std;

int main()

{

bool is_a_programmer=true;

cout << boolalpha << is_a_programmer << '\n';

cout << noboolalpha << is_a_programmer << '\n';

}

(4)

6.4.4	Escape Sequence

| Character | Description |
| --- | --- |
| \b | used to delete the previous character (backspace) |
| \n | is used to jump to the next line. |
| \t | Horizontal tab |
| \v | Vertical tab |
| \? | Question mark |
| \a | Beep sound |
| \’ | Single quote |
| \” | Double qoute |

(4)

[Total =12 marks]

# FORMATIVE ASSESSMENT 6.5			INDIVIDUAL TASK

6.5.1	Manipulators are functions specifically designed to be used in conjunction with the 	insertion (<<) and extraction (>>) operators on stream objects.			(2)

6.5.2	Manipulators with no arguments

- endl: A new line is entered, and the output stream is flushed after each new line.

- ends: It is also defined in ostream and it inserts a null character into the output stream.

- flush:  it forces all the output written on the screen or in the file.

(6)

6.5.3	Manipulators with arguments

setw() function allows programmer to set the minimum width of the next output via the insertion operator.

Syntax

cout<<setw(spaces)<<string/variable<<endl;

Example

cout<<setw(20)<<"We are developers"<<setw(20)<<"We are coders"<<endl;

Output

We are developers       We are coders

setfill()-It replaces setw(whitespaces )’s with a different character.

cout<< setfill('*') << endl;

cout<<setw(20)<<"We are developers"<<setw(20)<<"We are coders"<<endl;

***We are developers*******We are coders

setprecision(): The setprecision() utility is often used to alter the sequence of 	numbers displayed inside a floating-point integer output sequence

Example

double my_number=234.678;

cout<< setprecision(3)<<my_number<<endl;

Output

235

(9)

[Total =17 marks]

# SUMMATIVE ACTIVITY 6.6			INDIVIDUAL TASK

6.6.1 Data Types

| Item | Value | Data Type |
| --- | --- | --- |
| a | is_a_student=false; | bool |
| b | "We are learning about data types"; | string |
| c | 15; | int |
| d | 0b11; | int |
| e | L'x'; | wchar_t |
| f | 233434.56343; | double |
| g | 277.65 | float |
| h | 45e-2 | Float |

(8)

6.6.2 Swapping numbers without using a temporary variable.

#include <iostream>

using namespace std;

int main()

{

cout << " Swap two numbers :\n";

cout << "_____________________________\n";

int x1, x2;

cout << " Input 1st number : ";

cin >> x1 ;

cout << " Input 2nd number : ";

cin >> x2;

//swapping numbers

x1=x1+x2;

x2=x1-x2;

x1=x1-x2;

cout << " After swapping the 1st number is : "<<x1 <<"\n" ;

cout << " After swapping the 2nd number is : "<< x2<<"\n\n" ;

}

(10)

6.6.3

a) True

b) True											(2)

6.6.4	Assignment operators

|  | Simple expression | Expression using assignment operators |
| --- | --- | --- |
| i | a = a+2 ; | a += 2 ; |
| ii | a = a-3 ; | a -= 3 ; |
| iii | a = a*2 ; | a *= 2 ; |
| iv | a = a%2 ; | a %= 2 ; |
| v | b = b+(c+2); | b += c + 2 ; |
| vi | d =d (e * -5); | d *= e - 5 ; |

(6)

6.6.5	Both the ++x and x++ operators increase variable x by 1, but the ++x pre-increment 	operator follows the "change-then-use" philosophy while the x++ post-increment 	operator follows the "use-then-change" principle.					(4)

6.6.6	setbase()-Sets the basefield to one of its possible values: dec , hex or oct.

cout<< setbase(8)<<145<<endl;

cout<< setbase(16)<<145<<endl;

cout<< setbase(10)<<0b11<<endl;

OUTPUT

221

91

3

(8)

6.6.7	Complex numbers are the numbers that are expressed in the form of a+ib 	where, a,b 	are real numbers and 'i' is an imaginary number called “iota”.			(2)

6.6.8	Complex number multiplication

#include <iostream>

#include <complex>

using namespace std;

int main()

{

//declaring the complex number variables and assigning values

complex<double> complex_two(2.2);

complex<double> complex_three(3.6, 1.14);

//adding complex numbers

complex<double> complex_four =complex_two * complex_three;

//displaying output

cout <<"The answer for multiplying " << complex_two<< " x " <<complex_three<< " = "<< complex_four<<endl;

return 0;

}

OUTPUT

The answer for multiplying (2.2,0) x (3.6,1.14) = (7.92,2.508)

(10)

[Total =50 marks]

# Module 7:Math, Interactive Input, Constants and Errors

After you have completed this module, you should be able to :

- Explain the process that happens when the computer encounters the scanf or cin statement

- Use the C++ cin to read user input (numeric) and store the result in a variable

- Use the C++ cin object to read user input (single character) and store the result in a variable

- Define the term validation

- Explain why user input validation is important.

7.2.	C++ Errors

- Define the term syntax

- Define the term syntax error

- Identify and correct syntax errors in C++ code

- Explain the relationship between syntax and compile time errors

- Define the term typographical error

- Explain when a typographical error is a syntax error

- Define the term logic error

- List and explain three common side effects of logic errors

7.3.	Math Library

- Explain the concept of a preprogramed function

- Explain the need for preprogramed math functions

- Explain the purpose of the math.h header file

- Write C++ code that includes the header file math.h

- C++ Errors

- Define the term syntax error

- Identify and correct syntax errors in C/C++ code

- Define the term logic error

- List and explain three common side effects of logic errors

- Write C++ code that makes use of the following common mathematical functions

- abs(n)

- pow(n,n2)

- sqrt(n)

- sin(n)

- cos(n,n2)

- tan(n)

- List the mathematical functions covered and the return type they will produce

- Write C++ code containing complex mathematical expressions and math functions

7.4.	Validation

- Define the term validation

- Explain why user input validation is important

- Write C++ code that will validate numeric input ensuring the input is within a pre-defined range

- Write C++ code that will validate character input ensuring the expected input is one 	of 3 (max) characters

7.5.	Symbolic Constants

- Define the concept of a constant value

- List common constants found in everyday life

- Define the term magic numbers as used by programmers

- Explain the advantage of using a constant for magic numbers when programming

- Define the term symbolic name

- Write C++ code that uses the const declaration qualifier to mark a value as constant

FORMATIVE EXERCISES				GROUP/INDIVIDUAL TASKS

TASK 7.1: Program to calculate length of the edges of the cube, if its volume is equal to 125 cm3.

Solution

#include <iostream>

#include <cmath>

using namespace std;

int main()

{

double volume, side;

cout << "Enter volume of a cube :" << endl;

cin >>volume;

if(cin.fail())

{

cout << "Invalid age, re-enter: ";

}

else{

//calculating the side of the cube with a given volume

side=cbrt(volume);

cout<<"The side of a cube whose volume is "<<volume <<"cm3 is " <<side<<"cm"<<"\n";

}

return 0;

}

SAMPLE OUTPUT

Enter volume of a cube :

125

The side of a cube whose volume is 125cm3 is 5cm

TASK 7.2	:Write a C++ Program to raise any number X to power N.

Solution

/*  C++ Program to raise any number X to power N  */

#include<iostream>

#include<cmath>   //for pow() function

using namespace std;

int main()

{

int x,n,result;

cout<<"Enter value of X :: ";

cin>>x;

cout<<"\nEnter value of N :: ";

cin>>n;

if (cin.fail()){

cout<<"The input must be numbers"<<endl;

}

else{

if (n<1) {

result=1;

cout<<"\nThe Power of Number [ "<<x<<" ^ "<<n<<" ] = "<<result<<"\n";

}else{

result=pow(x,n);

cout<<"\nThe Power of Number [ "<<x<<" ^ "<<n<<" ] = "<<result<<"\n";

}

}

return 0;

}

Output

Enter value of X :: 2

Enter value of N :: 0

The Power of Number [ 2 ^ 0 ] = 1

Enter value of X :: 6

Enter value of N :: vbfbz

The input must be numbers

# FORMATIVE ASSESSMENT 7.1		INDIVIDUAL TASK

7.1.1	The value 3 will be printed out. While x is a float, it has been typecasted  to an int 	data type as it is stored in n variable.							(3)

7.1.2	Declaring variables and multiple accept using single cin statement.

string firstname="", surname="";

int age=0;

cin>>firstname>>surname>>age;

(4)

7.1.3	b)- '\n' and endl both are used to print new line but endl flushes the buffer after 	printing new line.									(1)

7.1.4	If the program does not have using namespace std; then when you write cout <<; you 	would have to put std::cout <<; same for other functions such as cin, endl etc.	(2)

7.1.5	The effect will be that only the last letter is displayed. Char is a data

char grade = 'Fingoland';

cout<<grade<<endl;

Output will be letter l.

(3)

[Total =13 marks]

# FORMATIVE ASSESSMENT 7.2		INDIVIDUAL TASK

7.2.1	A bug is an error in the program code.						(2)

7.2.2	Syntax errors are bugs in a program which results from code violating the rule of C++ 	writing techniques or languages.							(2)

7.2.3	Two errors:

- width will not be recognised because the comment for length is not closed.

- New line sequence character incorrect. / used instead of \.			(2)

7.2.4	Area of a shape with length 5cm and width 3cm

#include <iostream>

#include <cmath>

using namespace std;

int main()

{

int length=0;// this is the length

cout<<"Enter length "<<endl;

cin>>length;

cout<<"Enter width"<<endl;

int width=0;  /*this is the width*/

cin>>width;

int area=0;

area=length * width;

cout<<"The area of a shape whose length is " <<length<< " and width is "<<width<< " is " <<area<<"cm^2"<<"\n";

return 0;

}

Output

Enter length

5

Enter width

3

The area of a shape whose length is 5 and width is 3 is 15cm^2

(4)

7.2.5	Compile-Time Errors									(1)

7.2.6	Syntax error and Semantic errors.							(2)

7.2.7	Causes of runtime errors.

  - Division by 0

  - Invalid memory access during run-time.

  - Large allocation of memory together/Large Static Memory Allocation	(3)

[Total =16 marks]

# FORMATIVE ASSESSMENT 7.3		INDIVIDUAL TASK

7.3.1	<cmath> defines symbols in the std namespace, and may also define symbols in the 	global namespace whereas <math.h< defines symbols in the global namespace, and 	may also define symbols in the std namespace. math.h is deprecated in C++.	(4)

7.3.2	Completing the table								          (14)

| Function | Description | Example |
| --- | --- | --- |
| abs(x) | Returns the absolute value of x | abs(-9.5)=9 |
| pow(x) | returns the value of x to the power of y (xy): | pow(3,3)=27 |
| sqrt(x) | Returns the value of square root of x | sqrt(144)=12 |
| ceil(x) | rounds x number upwards to its nearest integer | ceil(8.9)=9 |
| floor(x) | rounds x number downwards to its nearest integer, and returns the result | floor(7.9)=7 |
| cbrt(x) | returns the cube root of the given argument | cbrt(64)=4 |
| remainder() | computes the floating-point remainder of numerator/denominator (rounded to nearest). | remainder(5/2.13) =0.74 |

7.3.3	Solving a mathematical expression using formula:

(a+b)^2

Values of a and b are entered by the user.

#include <iostream>

#include <cmath>

using namespace std;

int main(){

double a=0, b=0,   answer;

cout<<"Enter the value of a"<<endl;

cin>>a;

cout<<"Enter the value of b"<<endl;

cin>>b;

answer=pow(a,2)+2*a*b+ pow(b,2);

cout << "Answer is: " << answer << endl;

return 0;

}

OUTPUT

Enter the value of a

2.4

Enter the value of b

4.5

Answer is: 47.61

(10)

7.3.4	16											(2)

[Total =30 marks]

# FORMATIVE ASSESSMENT 7.4		INDIVIDUAL TASK

7.4.1	Validation or data validation means checking or verifying any data before it is 	allowed into a computer system.							(2)

7.4.2	Data validation ensures that incoming data is accurate, complete, and correct (valid).

Validation also helps ensure data was not compromised or corrupted during 	transmission.										(2)

7.4.3	Types of data validation

- Format validation

- Data type validation

- Range validation								(3)

7.4.4	A range check will verify whether input data falls within a predefined range. An 	example is when capturing the marks of students for a test in %. The allowable 	values must be between 0 and 100.							(3)

7.4.5	Range check and Numeric validation

#include <iostream>

using namespace std;

int main()

{

int mark=0;

cout << "Enter student test score:" << endl;

cin >> mark;

if(cin.fail())

{

cout << "This is not a valid mark. Entry must be a number " << endl;

}

else{

while(mark < 0 || mark> 100) {

cin.clear();

cin.ignore(1000, '\n');

cout << "Entry is outside the range, re-enter: ";

cin >> mark;

if (mark<51){

cout<<"Unfortunately. You failed the test"<<endl;

}

else{

cout<<"Congratulations. You passed the test"<<endl;

}

}}

return 0;

}

Sample Output

Enter student test score:

dgdgsfdg

This is not a valid mark. Entry must be a number

Enter student test score:

986

Entry is outside the range, re-enter: 56

Congratulations. You passed the test

(12)

[Total =22 marks]

# FORMATIVE ASSESSMENT 7.5		INDIVIDUAL TASK

7.5.1	Constants refer to fixed values that the program may not alter.			(2)

7.5.2	Declaring constants in C++.							            (2)

- Using const keyword

- Using #define header file

7.5.3	const int DISTANCE=257;								(2)

7.5.4	A magic number is a numeric literal that is used in the code without any explanation 	of its meaning.										(2)

7.5.5	Two advantages of using constants.

- Readability of the program: an identifier of a constant with a significant name is much more readable than a magic number.

- Modifiability of the program: -constants need to be modified only in the declaration if changes are required.						(4)

7.5.6	The program will print the word We.

Reason: The reason that only the word "We" is picked up in the message array is that 	the extraction operator >> ignores leading white space characters and then reads non-	white space characters up to the next white space character.				(3)

7.5.7	Volume of a sphere

#include <iostream>

#include <cmath>

using namespace std;

int main(){

int radius=0;

double volume=0;

const double PI=3.142;

cout << "Please enter the radius of the sphere:" << endl;

cin >> radius;

if(cin.fail()){

cout << "This is not a valid radius" << endl;

cin.clear();

cin.ignore(1000, '\n');

}

else{

volume= (4 *PI*pow(radius, 3))/3;;

cout<<"The volume of a sphere whose radius is " <<volume <<"cm^3"<<endl;

}

return 0;

}

Output

Please enter the radius of the sphere:

6

The volume of a sphere whose radius is 904.896cm^3

(10)

[Total =25 marks]

# SUMMATIVE ACTIVITY 7.6			INDIVIDUAL TASK

7.6.1	a) <<											(1)

7.6.2	b) >>											(1)

7.6.3	A runtime error in a program is an error that occurs while the program is running after 	being successfully compiled.								(2)

7.6.4	b) Compile-time error									(1)

7.6.5 setprecision()/ setw()									(2)

#include <iostream>

#include <iomanip>

using namespace std;

int main()

{

const double PI = 3.14159265;

cout<<setprecision(7)<<PI <<endl;

}

7.6.6 Area of a triangle using Heron’s formula

#include <iostream>

#include <cmath>

using namespace std;

int main(){

float side1, side2, side3, area, s;

cout << "\n\n Find the area of any triangle using Heron's Formula :\n";

cout << "-------------------------------------------------------\n";

cout<<" Input the length of 1st side  of the triangle : "<<endl;

cin>>side1;

cout<<" Input the length of 2nd side  of the triangle : "<<endl;

cin>>side2;

cout<<" Input the length of 3rd side  of the triangle : "<<endl;

cin>>side3;

//Input validation

if(!cin || cin.fail())

{

cout << "Invalid Please make sure all entries are in correct format"<<endl;

}

else

{

cout << "valid entry"<< endl;

s = (side1+side2+side3)/2;

area = sqrt(s*(s-side1)*(s-side2)*(s-side3));

cout<<" The area of the triangle is : "<< area << endl;

cout << endl;

return 0;}

}

OUTPUT

Find the area of any triangle using Heron's Formula :

----------------------------------------------------------

Input the length of 1st side  of the triangle :

9

Input the length of 2nd side  of the triangle :

5

Input the length of 3rd side  of the triangle :

8

valid The area of the triangle is : 19.8997cm^2

Find the area of any triangle using Heron's Formula :

----------------------------------------------------------

Input the length of 1st side  of the triangle :

9

Input the length of 2nd side  of the triangle :

100

Input the length of 3rd side  of the triangle :

2

Valid entry

The sides of the triangle are not valid :

(15)

7.6.7	A typographical error is a mistake made in the typing of keywords and statements or 	expressions. 										(2)

[Total=26marks]

# Module 8: Selection control structure

After you have completed this module, you should be able to :

- Define the term relational expression

- Explain the anatomy of a simple relational expression

- List and explain the different C++ relational operators

- Identify valid relational expressions

- Identify invalid relational expressions

- Explain the numeric result that is generated from a- relational expression

- Write C++ code that will save the result of relation expression that contains both or a mix of numeric variables and literals in an int variable

- Write C++ code that will save the result of relation expression that contains both or a mix of numeric variables and literals in a Boolean variable

- Explain how the integer result produced by a relation expression relates to bool true/false

- Write C++ code that will save the result of relation expression that contains both or a mix of character variables and literals in an int and bool variable

- Students should be able to

- Explain the purpose of the if statement in C++

- Explain the purpose of the else statement in C++

- Determine the program flow when an if statement is encountered

- Identify or correct the general form for an if-else statement

- Write C++ code that will use relational expressions in if-else statements (Max 2 two nested levels only)

- Write C++ code that will use relational expressions in if-else statements with compounded content (Max 2 two nested levels only)

- Explore the concept of block scope when writing if-else statements with compounded content

- Define the term logic operator

- List the different logic operators

- Explain how each logic operator will influence the result of an expression

- Write C++ code that will save the result of relation expression that contains both or a mix of numeric variables, character variables and literals and a single logical operator in an int variable

- Write C++ code that will save the result of relation expression that contains both or a mix of numeric variables, character variables and literals and a single logical operator in a bool variable

- Write C++ code that will save the result of relation expression that contains both or a mix of numeric variables, character variables and literals and multiple (Max 3) logical operator in a bool variable

- Write C++ code that will save the result of relation expression that contains both or a mix of numeric variables, character variables and literals and multiple (Max 3) logical operator in an int variable

- Define the term nested if statement

- Determine the application flow when a nested if statement is encountered

- Identify or correct the general form for a nested if-else statement

- Write C++ code that will use relational expressions in nested if-else statements 	(Max 3 levels)

- Write C++ code that will use relational expressions in nested if-else statements with 	compounded content (Max 3 levels)

- Write C++ code that will use relational expression containing logic operators in nested if-else statements (Max 3 levels, Max 3 logic operators per level)

- Write C++ code that will use relational expressions containing logic operator in nested if-else statements with compounded content (Max 3 levels, Max 3 logic

operators per level)

FORMATIVE EXERCISES				GROUP/INDIVIDUAL TASKS

TASK 8.1:

Write a C++ program to calculate the total cost of a product purchased in an electrical shop given the following conditions:

SOLUTION

#include <iostream>

int main()

{

using namespace std;

int quantity=0;

double discount, price=0;

cout << "Enter quantity" << endl;

cin >> quantity;

price = quantity*100;

if (price>1000){

cout<<"You qualify for a discount"<<endl;

cout<<"The total price without discount is R" <<price<<".00"<<endl;

discount = price * .1;

cout<<"The discount is " <<discount<<endl;

cout << "Total cost with discount subtracted is R" << price-(price*.1) <<".00"<< endl;

}

else{

cout << "Total cost is R" <<price << endl;

}

return 0;

}

OUTPUT

Enter quantity of items to be bought

15

You qualify for a discount

The total price without discount is R1500.00

The discount is 150

Total cost with discount subtracted is R1350.00

TASK 8.2

Write a program in C++ to check triangle is an isosceles, equilateral, or scalene using if-else statement. The program must allow the user to enter the size of the three sides.

SOLUTION

#include<iostream>

using namespace std;

int main()

{

//accepting input

int s1,s2,s3;

cout<<"please enter first side of the triangle"<<endl;

cin>>s1;

cout<<"please enter second side of the triangle"<<endl;

cin>>s2;

cout<<"please enter third side of the triangle"<<endl;

cin>>s3;

//evaluating the conditions

if(s1==s2 && s2==s3)

{

cout<<"This is an equilateral triangle"<<endl;

}

else if(s1==s2||s1==s3 || s2==s3)

{

cout<<"This is an isosceles triangle"<<endl;

}

else

{

cout<<"This is a scalene triangle";

}

}

Task 8.3

Write a C++ program which displays the number of days in a month captured by the user. The user enters an integer to represent the month. January is 1—December is 12.

Solution

#include <iostream>

using namespace std;

int main() {

int month;

cout<<"Enter Month in form of an integer between 1 and 12"<<endl;

cin>>month;

/* Input Validation */

if(month < 1 || month > 12){

cout<<"You entered an invalid input"<<endl;

return 0;

}

if(month == 2) {

cout<<" 28 or 29 Days in month "<<endl;

} else if(month == 4 || month == 6 || month == 9 ||

month == 11) {

cout<<" 30 Days in Month "<<endl;

} else if (month == 1 || month == 3 || month == 5 ||

month == 7 || month == 8 || month == 10 || month == 12) {

cout<<" 31 Days in month "<<endl;

}

return 0;

}

OUTPUT

Enter Month in form of an integer between 1 and 12

3

31 Days in month

Enter Month in form of an integer between 1 and 12

13

You entered an invalid input

Task 8.4

Program to ask a user to enter marks and print the corresponding grade using if… else if…else statements. Make sure the data is validated.

Solution

#include <iostream>

int main()

{

using namespace std;

int marks;

cout << "Enter marks" << endl;

cin >> marks;

if (cin.fail()){

cin.clear();

cin.ignore(1000, '\n');

cout << "The data entry failed to meet the requirements"<<endl;

return 0;

}

if (marks < 25 && marks >0){

cout << "F" << endl;

}

else if(marks >=25 && marks <45){

cout << "E" << endl;

}

else if(marks >=45 && marks <50){

cout << "D" << endl;

}

else if(marks >=50 && marks <60){

cout << "C" << endl;

}

else if(marks >=60 && marks <80){

cout << "B" << endl;

}

else if(marks >=80 && marks <100){

cout << "A" << endl;

}

else{

cout << "Invalid marks" << endl;

}

return 0;

}

TASK 8.5

Write a C++ program  to check that the entered character is small, capital or a special character.

SOLUTION

#include<iostream>

using namespace std;

int main()

{

char character;

cout<<"Enter a character =  ";

cin>>character;

int Ascii_value=character;

cout<<"The ASCII value  of "<<character<<

" is "<<Ascii_value;

if(Ascii_value>=97 && Ascii_value<=122)

{

cout<<"\nYou have entered a small letter";

}

else if(Ascii_value>=65 && Ascii_value<=90)

{

cout<<"\nYou have entered a capital letter";

}

else if (Ascii_value>=0 && Ascii_value>=47

|| Ascii_value>=54 && Ascii_value<=64

|| Ascii_value>=91 && Ascii_value<=96

|| Ascii_value>=123 && Ascii_value<=127)

{

cout<<"\nYou have entered a special character";

}

else if (Ascii_value>=47 && Ascii_value<=57)

{

cout<<"\nYou have entered a digit ";

}

return 0;

}

OUTPUT

Enter a character =  #

The ASCII value  of # is 35

Enter a character =  S

The ASCII value  of S is 83

You have entered a capital letter

# FORMATIVE ASSESSMENT 8.1		INDIVIDUAL TASK

8.1.1	A relational expression is a combination of two or more operands evaluated using an 	operator like a relational operator. Result of relational expression is boolean.	(2)

8.1.2	Relational Operators

- = =	Is equal to

- !=	not equal to

- > 	Greater than

- <	less than

- >= 	Greater than or equal to

- <= 	Less than or equal

ANY CORRECT FIVE							(5)

8.1.3	Examples of any THREE invalid relational operators

  - (x=>y)	invalid operator

  - (x=y)		Assignment operator instead of equality operator

  - (x=!y)	Invalid operator

  - true <= false										(3)

8.1.4	False											(1)

8.1.5	True											(2)

8.1.6	False											(2)

8.1.7	False											(2)

8.1.8	False											(2)

8.1.9	* 											(2)

8.1.10

#include <iostream>

using namespace std;

int main(){

int x = 7;

int y = 10;

if(y == x) {

cout<< "PASS"<<endl;  }

return 0;}

(2)

8.1.11	Printing True or False

#include <iostream>

using namespace std;

int main(){

int x = 7;

int y = 10;

int result=(x>y);

cout<<boolalpha<<(bool) result<<endl;

return 0;}

(4)

[Total = 27marks]

FORMATIVE ASSESSMENT 8.2		INDIVIDUAL TASK

8.2.1	Simple if statement

if (condition) {

statement(s)

}

(3)

8.2.2	Selection/conditional statements are used to select part of a program to be executed 	based on condition. They are also called branching statements.			(2)

8.2.3	The else statement allows the program to take alternative action/branching if the 	condition in if block is not met. When used with if---else if..else.

The statement takes a specific course of action if all conditions have not been met.	(2)

8.2.4	General syntax of an if…else statement.

if (test-expression)

{

True block of statements

}

else

{

False block of statements

}

Statements;

(4)

8.2.5	Triangle validation

#include<iostream>

using namespace std;

int main()

{

//accepting input

int s1,s2,s3;

cout<<"please enter first side of the triangle"<<endl;

cin>>s1;

cout<<"please enter second side of the triangle"<<endl;

cin>>s2;

cout<<"please enter third side of the triangle"<<endl;

cin>>s3;

if ((!(s1+s2>s3)) || (!(s1+s3>s2)) ||(!(s3+s2>s1))){

cout<<"not a valid triangle"<<endl;

}

else{

//evaluating the conditions

if(s1==s2 && s2==s3)

{

cout<<"This is an equilateral triangle"<<endl;

}

else if(s1==s2||s1==s3 || s2==s3)

{

cout<<"This is an isosceles triangle"<<endl;

}

else

{

cout<<"This is a scalene triangle";

}

return 0;

}

}

SAMPLE OUTPUT

please enter first side of the triangle

5

please enter second side of the triangle

89

please enter third side of the triangle

1

not a valid triangle

please enter first side of the triangle

7

please enter second side of the triangle

8

please enter third side of the triangle

8

This is an isosceles triangle

(15)

8.2.6	Displaying the maximum number from any three given numbers

#include <iostream>

using namespace std;

int main()  {

int num1, num2, num3, max_num;

cout<<"Enter Three Integers\n"<<endl;

cin>>num1>>num2>>num3;

if (cin.fail()){

cout<<"please check your inputs"<<endl;

return 0;     }

if(num1 > num2){

if(num1 > num3)

max_num = num1;

else

max_num= num3;

} else {

if(num2 > num3)

max_num= num2;

else

max_num = num3;    }

/* Print Maximum Number */

cout<<"The largest number from "<<num1<<","<<num2<<","<<num3<< " is "<< max_num<<endl;

return 0;}

OUTPUT

Enter Three Integers

5

9

7

The largest number is 9

Enter Three Integers

6

7

y

please check your inputs

(10)

8.2.7	Checking for bool entries

#include <iostream>

using namespace std;

int main ()

{

bool A,B;

cout<<"Please enter value of A"<<endl;

cin >>A ;

cout<<"Please enter value of B"<<endl;

cin >>B ;

if (cin.fail()){

cout << "Invalid entry"<<'\n';

return 0;}

if (A==1&&B==1)

{cout << "A And B"<<'\n';}

else if (A==1&&B==0)

{cout << "A or B"<<'\n';}

else {

{cout << "Invalid entry"<<'\n';}

}

return 0;

}

OUTPUT

Please enter value of A

1

Please enter value of B

0

A or B

(10)

[Total =46 marks]

# FORMATIVE ASSESSMENT 8.3		INDIVIDUAL TASK

8.3.1	True											(1)

8.3.2	Logical operators are used to check whether an expression is true or false.		(2)

8.3.3	Logical Operators

- &&  	Logical AND

- || 	Logical OR

- ! 	Logical NOT								(3)

8.3.4	A flag is a Boolean variable signalling that some condition exists in the program. When the flag is set to false it indicates the condition does not yet exist. When the flag is set to true it indicates that the condition does exist.					(2)

8.3.5	Write the C++ code snippet that increases the score by 5 marks if score is between 80 and 90.

if (score >= 80 && score <=90)

score += 5;

cout<<score<<endl;

(3)

8.3.6	C++ statement that prints true if x is an odd number and positive

if (x % 2 != 0 && x > 0)

{

cout<<true<<endl;

}

else{

cout<<false<<endl;

}

(3)

[Total =14 Marks]

# FORMATIVE ASSESSMENT 8.4		INDIVIDUAL TASK

8.4.1	Nested if in C++ is using more than one if statements in the same scope. The if statement is a decision-making statement that allows taking decisions based upon the condition specified.									(2)

8.4.2	Flowchart to show nested if statement.						(5)

8.4.3	Output of the program:

35 45 10

(2)

8.4.4	Outer Loop The evaluation of condition is always start from outer if. If the condition 	of outer if statement return true then inner if is evaluated.				(3)

8.4.5	C++ allows multiple levels of nested if statements.					(1)

[Total =13 Marks]

# SUMMATIVE ASSESSMENT  8.5		INDIVIDUAL TASK

8.5.1	When working with relation expression, C++ gives a 1 or 0 as output of the 	evaluation process. A one indicates true and a 0 represents false. These values can be 	changed to true or false by add the boolalpha statement before the result variable 	storing the output.									(2)

8.5.2	Checking for valid triangle

#include <iostream>

using namespace std;

int main() {

int angle1=0, angle2=0, angle3=0;

cout<<"Please enter the size of first angle"<<endl;

cin>>angle1;

cout<<"Please enter the size of second angle"<<endl;

cin>>angle2;

cout<<"Please enter the size of third angle"<<endl;

cin>>angle3;

if (cin.fail()){

cout<<"Invalid type entries"<<endl;

return 0;

}

if( angle1 + angle2 + angle3 == 180) {

cout<<"It is a Valid Triangle\n"<<endl;

} else {

cout<<"It is an invalid Triangle"<<endl;

}

return 0;

}

OUTPUT

Please enter the size of first angle

60

Please enter the size of second angle

85

Please enter the size of third angle

74

It is an invalid Triangle

Please enter the size of first angle

60

Please enter the size of second angle

60

Please enter the size of third angle

60

It is a Valid Triangle

(10)

8.5.3	The output is 0 which is same as false. This is because ASCII ‘a’ is 97  and ‘A’ is 65. 97<65 is false.										(3)

8.5.4	Nested if….else if …..

// Outer if condition
if (condition 1) {
   // inner if condition
   if (condition 2) {
       // Block of Code and Statements
   }
   // inner else condition
   else {
       // Block of Code and Statements
   }
}
// Outer else statement
else {
   // inner if condition
   if (condition 3) {
       // Block of Code and Statements
   }
   // inner else condition
   else {
       // Block of Code and Statements
   }
}

(4)

8.5.5	Calculating Profit/Loss

#include <iostream>

using namespace std;

int main() {

double costPrice=0, sellingPrice=0;

cout<<"Enter Cost Price "<<endl;

cin>>costPrice;

cout<<" Selling Price\n"<<endl;

cin>>sellingPrice;

if (cin.fail()){

cout<<"Entry type error"<<endl;

return 0;

}

if(costPrice > sellingPrice) {

/* Loss */

cout<<" You incurred a loss of = R"<<costPrice - sellingPrice<<endl;

} else if(sellingPrice > costPrice) {

/* Profit or Gain*/

cout<<"You made a profit of  =R"<< sellingPrice - costPrice<<endl;

} else {

/* No Profit or Loss*/

cout<<"No Profit and No Loss\n"<<endl;

}

return 0;

}

Enter Cost Price

100

Selling Price

200

You made a profit of  =R100

(10)

8.5.6

- If n has the value 7 , the output will be "

The number is positive”.

- If n has the value 15 , then there will be no output.

- If n has the value -3 then the output will be

"The number is ____________".

(6)

8.5.7

- The output will “be less than 10".						(2)

- The output will be " greater than 15".						(2)

- The output will be "less than 10".						(2)

- There is no value for n that will cause the output to be "not interesting".	(2)

That part of the code can never be executed!

[Total =43 Marks]

# Module 9:	Repetition control structure

After you have completed this module, you should be able to :

- Define the term pre-test loop and entrance-controlled loop

- Define the term fixed-count loop

- Define the term variable condition loop

- Explain the purpose of the while statement in C/C++

- Determine the application flow when a while statement is encountered

- Identify or correct the general form for a while statement

- Write C/C++ code that will use relational expressions in while statement. (Max 2 levels only)

- Write C/C++ code that will use relational expressions in while statements with compounded 	content (Max 2 two nested levels only)

- Explore the concept of block scope when writing while statements with compounded content

- Explain the difference between fixed-count loop and variable condition loop

- Write C/C++ code that will use relational expressions in while statements with interactive input (Max 2 two nested levels only)

- Explain what a break statement is used for

- Explain what the continue statement is used for

- Write C/C++ code that will use relational expressions in while statements making use of the break statement

- Write C/C++ code that will use relational expressions in while statements making use of the continue statement

- Write C/C++ code that will use relational expression containing logic operators in while (Max 3 level, Max 3 logic operators per level)

- Write C/C++ code that will use relational expressions containing logic operator in while loop with compounded content (Max 3 level, Max32 logic operators per level)

- Define the term post-test loop and exit controlled loop

- Determine the application flow when a do while statement is encountered

- Identify or correct the general form for a do while statement

- Define the term sentinel

- Explain where and for what reason a sentinel is used

- Write C/C++ code that will use relational expression containing logic operators in a do while loop (Max31 level, Max 3 logic operators per level)

- Write C/C++ code that will use relational expressions containing logic operator in a do while loop with compounded content (Max 3 level, Max 3 logic operators per level)

- Write C/C++ code that will use relational expression containing logic operators in a sentinel controlled do while loop (Max 3 level, Max 3 logic operators per level)

- Write C/C++ code that will use relational expressions containing logic operator in a sentinel controlled do while loop with compounded content (Max 3 level, Max 3 logic operators per level)

- Define the term variable condition loop

- Explain how the, for statement differs from the while

- Explain how the, for statement is similar to the while

- Identify or correct the general form for a for statement

- Explain the purpose of each part in the for-loop initialisation

- Explain what effect an empty semicolon will have on the for loop

- Explain the term infinite loop

- Write C++ code to create an infinite for loop.

- Use the break statement to end an infinite for loop based on the loop reaching a condition.

- Determine the application flow when for statement is encountered.

- Write C++ code that will use a for loop with a predetermined number of loops.

- Write C++ code that will use a for loop with a non-sequential counter variable.

- Define the term nested loop

- Define the terms inner and outer loops

- Write C++ code that will nest identical type loops (Max 3 levels)

- Write C++ code that will nest different type loops (Max 3 levels)

- Determine the application flow when nested loop statement is encountered

- Explain what a break statement is used for

- Explain what the continue statement is used for.

- Implement a break or continue statement as required as part of a code solution.

- Plan, Design, create and code a C++ solution incorporating various structures and nested 	strictest to solve a given problem.

FORMATIVE EXERCISE 						INDIVIDUAL TASKS

Task 9.1

Program to calculate factorial of a number requested by the user using a while… loop.

HINT: Factorial n! = n * (n-1) * (n-2) * (n-3) … *.

SOLUTION

#include <iostream>

using namespace std;

int main() {

int n;

cout<<"Please enter the number to be calculated"<<endl;

cin>>n;

int factorial = 1;

int i = 1;

if (cin.fail()){

cout<<"The value entered in not a number"<<endl;

}

else{

while (i<=n ) {

factorial = factorial* i;

i++;

}

cout << "Factorial of  " << n<< " is "<<factorial;

}

return 0;

}

OUTPUT

Please enter the number to be calculated

5

Factorial of  5 is 120

TASK 9.2

Program to generate Fibonacci numbers up to a number entered by the user

SOLUTION

#include <iostream>

using namespace std;

int main() {

//declaring and intialising variables

int first_num = 0;

int second_num = 1;

int next_num = 0;

int user_limit;

cout << "Enter a positive number : ";

cin >> user_limit;

if (cin.fail() || user_limit<0){

cout<<"Your input is invalid"<<endl;

goto ineligible;

}

// output

cout << "The following are the Fibonacci numbers between 0 and " <<user_limit<< endl;

cout<<first_num << ", " << second_num << ", ";

//calculating the third value

next_num = first_num + second_num;

//looping to generate the Fibonacci numbers

while(next_num <= user_limit) {

cout << next_num << ", ";

first_num = second_num;

second_num = next_num;

next_num = first_num + second_num;

}

return 0;

ineligible:

return 0;

}

OUTPUT

Enter a positive number : 150

The following are the Fibonacci numbers between 0 and 150

0, 1, 1, 2, 3, 5, 8, 13, 21, 34, 55, 89, 144,

TASK 9.3

Program to print a timestable- from 1 to 10 using nested while loops.

SOLUTION

#include <iostream>

#include <iomanip>

using namespace std;

int main()

{

int i = 1;

//Outer loop

while (i <=10) {

int j = 1;

cout <<"" << endl;

//Inner loop

while ( j <= 10) {

cout << i * j <<setw(5);

j++;

}

cout <<setw(5)<< endl;

i++;

}

return 0;

}

TASK 9.4

Program using a while loop to produce the following output:

54321

4321

321

21

1

SOLUTION

#include <iostream>

using namespace std;

int main(){

int i, number, inner;

cout << "Please enter number of line you want printed  ";

cin >> number;

i = number;

while (i >= 1)

{

cout << "\n";

inner = i;

while (inner >= 1)

{

cout << inner;

inner = inner - 1;

}

i = i - 1;

}

return 0;

}

TASK 9.5

Program using while loops to calculate salary of an employee whose rate and hours are entered by the user. The program must repeat the process and break when the number of hours entered by the user is equal to 0.

SOLUTION

#include <iostream>

#include <cmath>

#include <iomanip>

using namespace std;

int main() {

//declaring variables

double  hours, rate;

float salary=0;

while (true){

//accepting input

cout<<"Input your number of hours:";

cin>> hours;

//validating input

if (hours <=1 || cin.fail()){

cout<<"Unfortunately, incorrect entry or threshold not met to get a salary"<<endl;

break;

}

cout<<"Input your hourly rate:";

cin>> rate;

//processing

salary=hours * rate;

//formatting output

cout.precision(2);

cout<<"Your total salary is R"<<fixed<<salary<<endl;

}

cout<<"Good bye"<<endl;

return 0;

}

TASK 9.6

Program which prints all multiples of 5 from 100. If the value is divisible by3 and 5, skip the number.

Solution

//Program to show multiples of 3 from 30 to 3 excluding 15

#include <iostream>

using namespace std;

int i = 100;

int main() {

while (i > 0) {

if (i%5==0){

if (i%5==0 && i%3==0) {

i--;

continue;

}

cout << i << "\n";

}

i--;

}

return 0;

}

OUTPUT

100

95

85

80

70

65

55

50

40

35

25

20

10

5

TASK 9.7

Program using do loop to print small letters of the alphabet from ‘a’ to ‘z’.

SOLUTION

//Program to display alphabet characters from a to z

#include<iostream>

using namespace std;

int main()

{

//declare and initialise first letter

char letter='a';

do

{

cout<<letter<<" ";

letter++;

}

while(letter<='z');

cout<<"\nOut of the loop"<<endl;

return 0;

}

OUTPUT

a b c d e f g h i j k l m n o p q r s t u v w x y z

Out of the loop

TASK 9.8

Program to find the greatest common divisor of two numbers.

Solution

#include <iostream>

using namespace std;

int main()

{

int num1, num2, gcd;

cout << "\n\n Find the Greatest Common Divisor of two numbers:\n";

cout << "-----------------------------------------------------\n";

cout << " Input the first number: ";

cin >> num1;

cout << " Input the second number: ";

cin >> num2;

int i=1;

do{

if (num1 % i == 0 && num2 % i == 0)

{

gcd = i;

}

i++;

}while (  i <= num1 && i <= num2) ;

cout << " The Greatest Common Divisor is: " << gcd << endl;

return 0;

}

OUTPUT

Find the Greatest Common Divisor of two numbers:

-----------------------------------------------------

Input the first number: 88

Input the second number: 12

The Greatest Common Divisor is: 4

TASK 9.9

Program to implement a sentinel value using do loop to stop a user from continuing with transaction on the ATM.

SOLUTION

#include <iostream>

using namespace std;

int main()

{

char response;

cout<<"Would you like to do another transaction"<<endl;

cin>>response;

do{

cout<<"Withdrawal"<<endl;

cout<<"Statements"<<endl;

cout<<"Transfers"<<endl;

cout<<"Would you like to do another transaction"<<endl;

cin>>response;

}while(response!= 'N' );

cout << "Thank you for banking with us. enjoy your day" << endl;

return 0;

}

OUTPUT

Would you like to do another transaction

y

Withdrawal

Statements

Transfers

Would you like to do another transaction

Y

Withdrawal

Statements

Transfers

Would you like to do another transaction

N

Thank you for banking with us. enjoy your day

TASK 9.10

Program using do while loops in C++ to print a print a parallelogram or rhombus star pattern. as shown below.

*****

*****

*****

*****

*****

SOLUTION

#include<iostream>

using namespace std;

int main()

{

int i=1,j,k,n;

//outer loop

do

{

j=i;

do

{

cout<<" ";

j++;

}

while(j<=5);

k=1;

//inner loop

do

{

cout<<"*";

k++;

}

while(k<=5);

i++;

cout<<endl;

}

while(i<=5);

return 0;

}

TASK 9.11

Program to generate a 2 times table using a for loop statement.

Solution

#include <iostream>

using namespace std;

int main()

{

int answer=0;

cout << "\t\t2 Times Table\n";

cout << "-----------------------------------------------------\n";

//  initialisation expression, test condition & update expression in loop header

for (int i=1;i<=10;i++)

{

//processing

answer = i * 2;

//displaying the solution

cout<<i << " x "<<2<<" = "<< answer<<endl;

}

return 0;

}

Task 9.12

Program using for loops to calculate the factorial of a number entered by a user. Validate the input to allow positive numbers. If the value entered is not a number or less than 0, the program must give display an error message.

SOLUTION

#include<iostream>

#include<conio.h>

using namespace std;

int main()

{

int i;

int num;

unsigned long long fact=1;

cout<<"Please enter a number: ";

//accepting input

cin>>num;

//validating input

if (cin.fail() or num<0){

cin.clear();

cin.ignore(1000, '\n');

cout << "The data entry failed to meet the requirements"<<endl;

}

else{

//calculating factorial of a number

for(i=1;i<=num;i++)

{

fact=fact*i;

}

cout<<"Factorial of entered number is: "<<fact;

return 0;

}

}

TASK 9.13

Program using for loop which will loop from 1 to 20 and print the values. The must replace all occurrences of number divisible by 3 with the word FIZZ and occurrences of numbers divisible by 5 with the word BUZZ and all occurrences of numbers divisible by both 3 and 5 with the word FIZZ BUZZ.

SOLUTION

#include <iostream>

using namespace std;

int main()

{

// declare variables

int fizz = 3;

int buzz = 5;

int end_value = 20;

int fizzBuzz = fizz * buzz;

int counter1=0, counter2=0, counter3=0;

// ^ numbers divisible by 3 and 5 are also divisible by 3 * 5

// start the loop, continue until the counter (start_value) reaches the 'end_value

for (int start_value = 1; start_value <= end_value; ++start_value)

{

if (start_value % fizzBuzz == 0) // divisible by 3 and 5

{

cout<<"FIZZ BUZZ!!\n";

counter3=counter3+1;

counter2=counter2+1;

counter1=counter1+1;

}

else if (start_value % fizz == 0) // divisible by 3

{

cout<<"FIZZ\n";

counter1=counter1+1;

}

else if (start_value % buzz == 0)// divisible by 5

{

cout<<"BUZZ\n";

counter2=counter2 +1;

}

else // not divisible by 3 or 5

{

cout<<start_value<<endl;

}

}

//displaying the occurences of each word

cout<<"The number of occurences of the word FIZZ is "<<counter1<<endl;

cout<<"The number of occurences of the word BUZZ is "<<counter2<<endl;

cout<<"The number of occurences of the word FIZZBUZZ is "<<counter3<<endl;

return 0;

}

OUTPUT

1

2

FIZZ

4

BUZZ

FIZZ

7

8

FIZZ

BUZZ

11

FIZZ

13

14

FIZZ BUZZ!!

16

17

FIZZ

19

BUZZ

The number of occurences of the word FIZZ is 6

The number of occurences of the word BUZZ is 4

The number of occurences of the word FIZZBUZZ is 1

TASK 9.14

Write your own program in C++ to create an infinite loop using for loop.

SOLUTION

Students will create different programs.

TASK: 9.15

Program to print the following stars and patters:

******

*****

****

***

**

*

SOLUTION

#include<iostream>

using namespace std;

int outer =6;

int main()

{

while (outer>=1)

{

for(int s=1; s<=6-outer; s++)

cout<<" ";

for (int innermost=1; innermost<=outer; innermost++)

{

cout<<"*";

}

cout<<endl;

outer--;

}

}

# FORMATIVE ASSESSMENT 9.1		INDIVIDUAL TASK

9.1.1	A pre-test loop is a repetition structure where the condition of the loop is upfront in 	the loop declaration. 									(2)

9.1.2	B.	Infinite Loop									(1)

9.1.3	B.	Counter Variable								(1)

9.1.4	C.	Iteration									(1)

9.1.5	A.	It cancels remaining iterations and causes the code to exit the loop but can still 		any other code outside the loop.						(1)

9.1.6	B.	The innermost loop								(1)

9.1.7	The condition is always false. So ultimately, no statements will ever be executed in 	the body of the loop.									(2)

9.1.8

//Program to calculate power of a integer number

#include <iostream>

using namespace std;

//Declaring variables

int main() {

unsigned int i=1,result =1;

int base, exponent;

cout<<"Please enter the base of a number"<<endl;

cin>>base;

cout<<"Please enter the number which you are raising by (exponent)"<<endl;

cin>>exponent;

//validating input

if(cin.fail() || exponent<1 || base<1){

cout<<"Incorect entry"<<endl;

return 0;}

else{//processing

while (i<=exponent){

result= result * base;

++i;

}

//Displaying the output

cout<<base <<" raised to the power of " << exponent<< " = " <<result<<endl;}

return 0;

}

OUTPUT

Please enter the base of a number

4

Please enter the number which you are raising by (exponent)

5

4 raised to the power of 5 = 1024

(10)

9.1.9	The statement will never be printed. This is because the statement is never true 	because of the following line:

i = i - 1 ;

If the line is removed, the program will generate an infinite loop.			(2)    [Total = 21 Marks]

# FORMATIVE ASSESSMENT 	9.2		INDIVIDUAL TASK

9.2.1	Identify and correct the error(s) in each of the following statements given below:

a)	The letter W in while statement should be lowercase. The while’s body should be 	enclosed in braces because there is more than one statement.			(2)

- The value of y must be decremented otherwise, the code will generate an infinite loop.											(2)

9.2.2	Rewrite the following code fragment so that it uses a "while..." loop to accomplish the 	same task.

#include <iostream>

using namespace std;

int main()

{

int n;

cout << "Enter a non-negative integer: ";

cin>> n;

while (n < 0)

{

cout << "The integer you entered is negative." << endl;

cout << "Enter a non-negative integer: ";

cin>> n;

}

return 0;

}

OUTPUT

Enter a non-negative integer: -9

The integer you entered is negative.

Enter a non-negative integer: -5

The integer you entered is negative.

Enter a non-negative integer: 5

(10)

[Total =14 marks]

# FORMATIVE ASSESSMENT 	9.3		INDIVIDUAL TASK

9.3.1	C. do…while loop									(1)

9.3.2	C. do-while										(1)

9.3.3	do…loop										(3)

do{

//code to be executed

} while(condition);

(3)

9.3.4	break statement in a do loop cancels remaining iterations and exits from the loop but 	any other statements after the loop will be executed.					(2)

}

9.3.5	The answer is 32. do { } block is executed even before checking while(condition) at 	least once. This prints 32. To loop for the second time, while (32 <= 30) fails. So, 	loop 	is quit.										(3)

TOTAL=13 MARKS

# FORMATIVE ASSESSMENT 	9.4		INDIVIDUAL TASK

9.4.1	A. for(initialization; condition; increment/decrement)				(1)

9.4.2	The output will be: 12344								(1)

9.4.3	For loops are used when the number of iterations is known, whereas while loops are 	used when the user only knows the condition at which the program must stop/exit the 	loop.											(4)

9.4.4	if we place a semicolon after nothing, you will create a statement. Thus, the 	semicolon by itself is a statement, which is called the empty statement or the null 	statement. For 	example, the following statement will do nothing:

for(;;);{

cout<<"NCV";

}

However, if we remove the semicolon after the loop declaration, we will generate an 	endless loop printing the word NCV.							(4)

9.4.5	A break statement terminates the loop and a continue statement will skip a statement 	and then carry on with execution of loop statements until the end of the loop as 	specified by the test condition. Here is an example:

break statement

for(int i=0; i<=10; i++){

if (i==4){

break;

}

cout<<i;

}

OUTPUT

0123

continue statement

for(int i=0; i<=10; i++){

if (i==4){

continue;

}

cout<<i;

}

OUTPUT

01235678910

NB when break is used, the program terminates after printing up to 3. When continue 	is used the program prints 0 to 10 except 4.						(6)

9.4.6	Program to print multiples of 3 from 50 down.

#include <iostream>

using namespace std;

int main()

{

for (int i=50; i>0; i--){

if (i%3==0){

cout<<i<< " ";

}

}

return 0;

}

(5)

TOTAL =21 MARKS

# FORMATIVE ASSESSMENT 9.5 		INDIVIDUAL TASK

9.5.1

# FORMATIVE ASSESSMENT 9.6 		INDIVIDUAL TASK

9.6.1	Grading program using while loop and break statement.

#include <iostream>

using namespace std;

int main(){

int studentID, testScore, counter=0;

cout << "Enter the student ID and the test score . " << endl;

cout<< "To terminate the program enter -1 for the student ID ." << endl;

cout<<"Enter the student ID "<<endl;

cin>>studentID;

while (studentID != -1){

counter++;

cout<<"Please enter the test score " << endl;

cin>> testScore;

if (testScore<0){

cout<< "Negative number found in the data. Exiting " << endl;

break;

return 0;    }

cout << "Student ID = " << studentID << ",  test score =     " <<testScore << " ,  and grade = ";

if (testScore>=90){

cout<<"A grade." <<endl;

}

else if (testScore>=80){

cout<<"B grade." <<endl;

}

else if (testScore>=70){

cout<<"C grade." <<endl;

}

else if (testScore>=60){

cout<<"D grade." <<endl;

}

else if (testScore>=50){

cout<<"E grade." <<endl;

}

else{

cout<<"F grade." <<endl;

}

cout<<"The number of student in class = " << counter<<endl;

cout<<"Enter the student ID " << counter<<endl;

cin>>studentID;

}

return 0;

}

TOTAL =15 MARKS

# SUMMATIVE ASSIGNMENT		INDIVIDUAL WORK

9.7.1	C										(1)

9.7.2	B. 1 2 3 4 5									(1)

9.7.3	A	for								            (1)

9.7.4	C										(1)

    - C do…while loop								(1)

9.7.6	C	for(int i = 0; i<=10; i++);						(1)

9.7.7	C 1										(1)

9.7.8	Solution

/**

* C++ program to print all alphabets from a to z lowercase

*/

#include<iostream>

using namespace std;

int main()

{

//declaring variable

char ch;

cout<<"Alphabets from a - z are: \n";

//Loop with starting value, test expression and incrementor

for(ch='a'; ch<='z'; ch++)

{

//printing the results

cout<<" "<<ch;

}

return 0;

}

(10)

9.7.9	While…loop is a pretest loop and do… loop is a post test loop. The structure is 	different:										(2)

while (expression)
{statements;}

do {statement;}

while(condition);

(2)

9.7.10	Solution: Program to print a number in reverse

/**

* C program to find reverse of a number

*/

#include <iostream>

using namespace std;

int main()

{

int num, reverse = 0;

/* Input a number from user */

cout<<"Enter any number to find reverse: ";

cin>>num;

/* Repeat the loop until value of num== 0 */

while(num != 0)

{

/*

* Increase place value of reverse and

* add last digit to reverse

*/

reverse = (reverse * 10) + (num % 10);

/* Remove last digit from 'num' */

num =num/ 10;

}

cout<<"Reverse number is  : "<< reverse<<endl;

return 0;

}

OUTPUT

Enter any number to find reverse: 951

Reverse number is  : 159

(10)

TOTAL =31 MARKS

# Topic 10:	Modularisation and	Functions

# FORMATIVE ASSESSMENT 		INDIVIDUAL TASK

10.1.1	Scope is the area in the program through which a variable or identifier is valid in the 	program.										(2)

10.1.2	Local scope is a characteristic in programming  where the variables are local to the 	module or subroutine in which they have been declared. In the code below, variables 	salary and gross are local to the function salary_calc().

#include <iostream>

using namespace std;

//function prototype

void salary_calc();

int main()

{

salary_calc();

return 0;

}

//function

void salary_calc(){

double salary,gross;

}

(5)

[Total=7 marks]

# FORMATIVE ASSESSMENT 			INDIVIDUAL TASK

10.2.1	A   											(1)

10.2.2	D  											(1)

10.2.3	B 											(1)

10.2.4	D 											(1)

10.2.5

first = 8

second = 2

first = 9										(3)

10.2.6	A user-defined function is a function created by the programmer where he or she 	groups code to perform a specific task. User-defined functions are assigned names by 	the programmer.									(2)

10.2.7

i.	The function was declared after the main() function so will produce an error during compilation. A function prototype needs to be declared before the main program.	(4)

ii.	4, 4 											(2)

10.2.8	Programmers use functions to break down problems into smaller chunks, each performing a specific function.							(2)

10.2.9	Modularisation is a technique for developing software that involves grouping related 	code that is needed to solve a small problem within the larger problem, making it easier 	to extend, reuse, and debug.								(2)

10.2.10 True or False.										(5)

i.	FALSE

ii.	TRUE

    - TRUE

    - FALSE

10.2.11

SOLUTION

#include <iostream>

#include <iomanip>

#include <string>

using namespace std;

//function prototype

void salary_calc();

int main()

{

salary_calc();

return 0;

}

//function

void salary_calc(){

double salary,gross,ha,da;

cout<<"enter the basic salary of the employee."<<endl;

cin>>salary;

bool check_sal=isdigit(salary);

if (!cin.fail() && check_sal==1){

if(salary<= 10000 && salary>0.00)

{

da=salary*20/100;

ha=salary*80/100;

gross=salary+da+ha;

cout<<setprecision(2)<<fixed;

cout<<"the gross salary of the employee is R"<<gross;

}

else if(salary<= 20000 && salary >10000)

{

da=salary*25/100;

ha=salary*90/100;

gross=salary+da+ha;

cout<<setprecision(2)<<fixed;

cout<<"the gross salary of employee is R"<<gross;

}

else if(salary>20000)

{

da=salary*30/100;

ha=salary*95/100;

cout<<setprecision(2)<<fixed<<endl;

gross=salary+da+ha;

cout<<"the gross salary of employee is R"<<setprecision(2)<<gross<<"\n";

}

else

{

cout<<"you have no salary"<<endl;

}

}

else{

cout<<"Salary input not captured correctly"<<endl;

}

}

(15)

[TOTAL =41 marks]